Name – parikh marmik

VLAN and Static Routing Configuration Assignment

# 1. Create Two VLANs (Sales and Support)

## Objective

Create two VLANs on a Cisco switch.

- VLAN 10 – Sales

- VLAN 20 – Support

### Cisco CLI Commands

enable
configure terminal

vlan 10
name Sales
exit

vlan 20
name Support
exit

end
write memory

# 2. Assign Ports to VLANs and Verify

## Port Assignment

| Switch Port | VLAN | Department |
| --- | --- | --- |
| Fa0/1 | VLAN 10 | Sales |
| Fa0/2 | VLAN 10 | Sales |
| Fa0/3 | VLAN 10 | Sales |
| Fa0/4 | VLAN 20 | Support |
| Fa0/5 | VLAN 20 | Support |

## Configuration Commands

### Assign Ports Fa0/1–Fa0/3 to VLAN 10

enable
configure terminal

interface range fa0/1-3
switchport mode access
switchport access vlan 10
exit

### Assign Ports Fa0/4–Fa0/5 to VLAN 20

interface range fa0/4-5
switchport mode access
switchport access vlan 20
exit

end
write memory

## Verify VLAN Membership

show vlan brief

### Expected Output

| VLAN | Name | Assigned Ports |
| --- | --- | --- |
| 10 | Sales | Fa0/1, Fa0/2, Fa0/3 |
| 20 | Support | Fa0/4, Fa0/5 |

# 3. Configure Static Routing Between Two Routers

## Network Diagram

LAN 1
     192.168.1.0/24
           |
        Router 1
   G0/0:192.168.1.1
   G0/1:10.0.0.1
           |
=========================
 Ethernet/Serial Link
=========================
           |
   G0/1:10.0.0.2
        Router 2
   G0/0:192.168.2.1
           |
      LAN 2
   192.168.2.0/24

## Router 1 Configuration

enable
configure terminal

interface g0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit

interface g0/1
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end
write memory

## Router 2 Configuration

enable
configure terminal

interface g0/0
ip address 192.168.2.1 255.255.255.0
no shutdown
exit

interface g0/1
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end
write memory

## Verification

show ip route

Test connectivity:

ping 192.168.2.1

If configured correctly, the ping should be successful.

# 4. Ping Test Between Sales VLAN and Support VLAN

## Network Setup

| Device | VLAN | IP Address |
| --- | --- | --- |
| PC1 | Sales (VLAN 10) | 192.168.10.10 |
| PC2 | Support (VLAN 20) | 192.168.20.10 |

## Ping Test

From PC1 (Sales VLAN)

ping 192.168.20.10

### Result

Request timed out.

## Why Does the Ping Fail?

The ping fails because VLAN 10 and VLAN 20 are separate broadcast domains. A Layer 2 switch forwards traffic only within the same VLAN and cannot route packets between different VLANs.

## How to Allow Inter-VLAN Communication?

Inter-VLAN communication can be enabled by using:

- Router-on-a-Stick

  - Configure a trunk link between the switch and router.

  - Create subinterfaces on the router for each VLAN.

  - Assign IP addresses as the default gateways.

OR

- Layer 3 Switch

  - Create Switch Virtual Interfaces (SVIs) for each VLAN.

  - Enable routing using the ip routing command.

With either method, devices in different VLANs can communicate successfully.