# Assignment: VLANs, Trunking and Inter-VLAN Routing

## 1. Create a Network with Three Switches and Two VLANs

### Objective

To create a network with three switches and two VLANs:

- VLAN 10 – Sales

- VLAN 20 – Support

Each switch will have two PCs, with one PC assigned to VLAN 10 and one PC assigned to VLAN 20.

### Network Topology

Router

G0/0 Trunk

|

Switch1

/      \

PC1        PC2

VLAN 10     VLAN 20

Sales      Support

|

|

Trunk

|

Switch2

/     \

PC3      PC4

VLAN 10   VLAN 20

Sales     Support

|

|

Trunk

|

Switch3

/     \

PC5      PC6

VLAN 10   VLAN 20

Sales     Support

### VLAN Assignment

| Switch | PC | Port | VLAN | Department |
| --- | --- | --- | --- | --- |
| Switch1 | PC1 | Fa0/2 | 10 | Sales |
| Switch1 | PC2 | Fa0/3 | 20 | Support |
| Switch2 | PC3 | Fa0/2 | 10 | Sales |
| Switch2 | PC4 | Fa0/3 | 20 | Support |
| Switch3 | PC5 | Fa0/2 | 10 | Sales |
| Switch3 | PC6 | Fa0/3 | 20 | Support |

# 2. Create VLANs and Configure Trunk Ports

## Step 1: Create VLANs on Switch1

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

## Step 2: Create VLANs on Switch2

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

## Step 3: Create VLANs on Switch3

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

# Assign Access Ports

## Switch1

### PC1 → VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

### PC2 → VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

## Switch2

### PC3 → VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

### PC4 → VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

## Switch3

### PC5 → VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

### PC6 → VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

# Configure Trunk Links

Suppose:

- Switch1 Fa0/1 → Switch2 Fa0/1

- Switch2 Fa0/4 → Switch3 Fa0/1

- Switch1 Fa0/5 → Router

## Switch1

interface range fastEthernet 0/1, fastEthernet 0/5

switchport mode trunk

exit

## Switch2

interface range fastEthernet 0/1, fastEthernet 0/4

switchport mode trunk

exit

## Switch3

interface fastEthernet 0/1

switchport mode trunk

exit

### Verify Trunking

On each switch:

show interfaces trunk

The configured trunk ports should appear in the output.

### Example Screenshot Output

Port        Mode        Encapsulation    Status

Fa0/1       on          802.1q            trunking

Fa0/4       on          802.1q            trunking

Assignment requirement: Take a screenshot of the actual show interfaces trunk output from your Packet Tracer and attach it here.

# 3. Configure Router-on-a-Stick for Inter-VLAN Routing

### Objective

To allow devices in VLAN 10 and VLAN 20 to communicate using a router.

The router will use two sub-interfaces:

G0/0.10 → VLAN 10

G0/0.20 → VLAN 20

### Router Configuration

First enable the physical interface:

enable

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

## VLAN 10 Sub-Interface

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

## VLAN 20 Sub-Interface

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

### Router IP Address Table

| Sub-Interface | VLAN | IP Address |
| --- | --- | --- |
| G0/0.10 | 10 | 192.168.10.1 |
| G0/0.20 | 20 | 192.168.20.1 |

# 4. Configure IP Addresses on PCs

## VLAN 10 – Sales

### PC1

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

### PC3

IP Address: 192.168.10.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

### PC5

IP Address: 192.168.10.30

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

## VLAN 20 – Support

### PC2

IP Address: 192.168.20.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.20.1

### PC4

IP Address: 192.168.20.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.20.1

### PC6

IP Address: 192.168.20.30

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.20.1

### Complete IP Table

| PC | VLAN | IP Address | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 10 | 192.168.10.10 | 192.168.10.1 |
| PC3 | 10 | 192.168.10.20 | 192.168.10.1 |
| PC5 | 10 | 192.168.10.30 | 192.168.10.1 |
| PC2 | 20 | 192.168.20.10 | 192.168.20.1 |
| PC4 | 20 | 192.168.20.20 | 192.168.20.1 |
| PC6 | 20 | 192.168.20.30 | 192.168.20.1 |

# 5. Test Inter-VLAN Connectivity

### Test from PC1 to PC2

PC1 belongs to VLAN 10 and PC2 belongs to VLAN 20.

On PC1:

ping 192.168.20.10

### Expected Result

Reply from 192.168.20.10

Reply from 192.168.20.10

Reply from 192.168.20.10

This confirms that inter-VLAN routing is working.

### Test Other PCs

From PC3:

ping 192.168.20.20

From PC5:

ping 192.168.20.30

The pings should be successful.

# Troubleshooting if Ping Fails

If PC1 cannot ping PC2, check the following items.

## 1. Check VLAN Assignment

On the switches:

show vlan brief

Verify that:

PC1 → VLAN 10

PC2 → VLAN 20

## 2. Check Trunk Ports

Use:

show interfaces trunk

Make sure the links between the switches are trunk ports.

Also make sure the switch port connected to the router is a trunk.

## 3. Check Router Sub-Interfaces

Use:

show ip interface brief

Expected:

G0/0.10    192.168.10.1    up

G0/0.20    192.168.20.1    up

Also verify:

show running-config

Check that the VLAN numbers match:

G0/0.10

encapsulation dot1Q 10

G0/0.20

encapsulation dot1Q 20

## 4. Check PC Default Gateway

PC1:

Default Gateway = 192.168.10.1

PC2:

Default Gateway = 192.168.20.1

A wrong default gateway can prevent inter-VLAN communication.

# Final Verification

Use the following commands:

### Switch

show vlan brief

show interfaces trunk

### Router

show ip interface brief

show running-config

### PC

ping 192.168.20.10

### Expected Result

PCs from VLAN 10 should be able to communicate with PCs in VLAN 20 through the router.