Name – parikh marmik

Troubleshooting network services

# 1. Simulate a DNS Resolution Failure

### Objective

Redirect www.spotify.com to an incorrect IP address using the local hosts file.

### Steps

- Open the hosts file with administrator privileges.

  - Windows: C:\Windows\System32\drivers\etc\hosts

  - Linux: /etc/hosts

- Add the following entry:

192.168.1.250    www.spotify.com

- Save the file.

- Flush the DNS cache.

Windows

ipconfig /flushdns

Linux

sudo systemd-resolve --flush-caches

- Test using:

ping www.spotify.com

### Sample Output

Pinging www.spotify.com [192.168.1.250] with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.1.250:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

### Observation

The system resolves www.spotify.com to the incorrect IP address specified in the hosts file, causing the ping request to fail because no Spotify server exists at that address.

After the test: Remove the incorrect hosts entry and flush the DNS cache again to restore normal name resolution.

# 2. Incorrect Default Gateway in DHCP

### DHCP Configuration

| Parameter | Configured Value |
| --- | --- |
| DHCP Pool | 192.168.10.100–192.168.10.150 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.20.1 (Incorrect) |

### Client Configuration

| Item | Value |
| --- | --- |
| Assigned IP | 192.168.10.100 |
| Assigned Gateway | 192.168.20.1 |

### Symptoms

- The client receives an IP address successfully.

- The client cannot reach external networks or the internet.

- Local communication within the subnet may still work.

### How to Identify the Issue

- Check the client's IP configuration (ipconfig or ifconfig).

- Verify the assigned default gateway.

- Attempt to ping the gateway.

- Compare the configured gateway with the router's actual LAN IP address.

### Solution

Update the DHCP server so the default gateway matches the router's LAN address (for example, 192.168.10.1). Renew the client's DHCP lease and verify that the correct gateway is assigned.

# 3. DNS Troubleshooting Using nslookup

### Scenario

Devices can access websites using IP addresses but cannot access them using domain names.

### Diagnosis

Run:

nslookup www.google.com

### Example Problem

DNS request timed out.

Server: Unknown

Address: 192.168.10.20

### Troubleshooting Steps

- Verify the DNS server configured on the client.

- Ping the configured DNS server to confirm network connectivity.

- Test another DNS server (for example, 8.8.8.8).

- Check whether the DNS service is running.

- Verify DNS zone and forwarding configuration if using a local DNS server.

- Flush the DNS cache and test again.

### Solution

The client was configured with an incorrect or unavailable DNS server. Updating the DHCP configuration (or manually configuring a valid DNS server) restores domain name resolution.

# 4. Client Receives an APIPA Address (169.254.x.x)

### Symptoms

Example:

IPv4 Address : 169.254.25.18

Subnet Mask  : 255.255.0.0

Gateway      : None

### Possible Causes

- Network cable disconnected or faulty.

- Switch port is disabled.

- DHCP server is offline.

- DHCP service has stopped.

- DHCP address pool is exhausted.

- VLAN or network configuration is incorrect.

- Firewall is blocking DHCP traffic.

- DHCP relay agent is missing in routed networks.

### Troubleshooting Process

- Check physical network connectivity.

- Verify switch port status.

- Confirm the DHCP server is powered on and running.

- Check whether addresses remain available in the DHCP scope.

- Verify VLAN assignments and routing.

- Renew the IP address:

- ipconfig /release

ipconfig /renew

- Test DHCP from another client.

- Review DHCP server logs for errors.

### Solution

After restoring DHCP service (or expanding the DHCP scope if it was exhausted) and renewing the client's lease, the client receives a valid IP address from the DHCP server.

# 5. Common DNS and DHCP Issues

## DNS Issues

| Issue | Symptoms | Possible Cause | Recommended Solution |
| --- | --- | --- | --- |
| Incorrect DNS Server | Domain names do not resolve | Wrong DNS server configured | Configure a valid DNS server address |
| Incorrect DNS Record | Website opens the wrong server or fails | Wrong A or CNAME record | Correct the DNS record and reload the zone |
| DNS Service Stopped | All DNS queries fail | DNS server process is not running | Restart the DNS service and verify logs |

## DHCP Issues

| Issue | Symptoms | Possible Cause | Recommended Solution |
| --- | --- | --- | --- |
| DHCP Scope Exhausted | New devices cannot obtain an IP address | No addresses left in the pool | Increase the pool size or shorten lease times |
| Incorrect Default Gateway | Clients receive an IP but cannot reach other networks | Wrong gateway configured in DHCP | Update the DHCP gateway option and renew leases |
| DHCP Server Unavailable | Clients receive an APIPA (169.254.x.x) address | DHCP server is offline or unreachable | Restore DHCP service, verify connectivity, and renew the client lease |