# Assignment: MAC Address Table and Switch Forwarding

## 1. Observe an Empty MAC Address Table

### Objective

To create a simple network with one switch and two PCs and observe how the switch learns MAC addresses.

### Network Topology

PC1

|

|

Switch

|

|

PC2

### Steps

- Open Cisco Packet Tracer.

- Add one switch, for example Cisco 2960.

- Add two PCs.

- Connect PC1 and PC2 to the switch using Copper Straight-Through cables.

- Assign IP addresses.

### IP Address Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 |
| PC2 | 192.168.1.20 | 255.255.255.0 |

### Check MAC Address Table

Open the switch CLI and enter:

enable

show mac address-table

Before the PCs communicate with the switch, the MAC address table may have no dynamically learned PC MAC addresses.

### Observation

The switch does not need to know all MAC addresses beforehand. It learns them when frames arrive through its ports.

# 2. Send Ping and Observe MAC Address Learning

### Objective

To observe how the switch learns the MAC addresses of PCs after communication takes place.

From PC1, open Command Prompt:

ping 192.168.1.20

PC1 sends packets toward PC2.

Now go to the switch CLI and enter:

enable

show mac address-table

The switch should now show dynamically learned MAC addresses.

### Example

VLAN    MAC Address          Type       Port

----    -----------------    --------   ----

1       AAAA.BBBB.CCCC       DYNAMIC    Fa0/1

1       DDDD.EEEE.FFFF       DYNAMIC    Fa0/2

The exact MAC addresses will be different in your Packet Tracer file.

### What Happened?

When PC1 sent a frame, the switch saw PC1's source MAC address and learned:

PC1 MAC → Fa0/1

When PC2 replied, the switch learned:

PC2 MAC → Fa0/2

### Test in the Other Direction

From PC2:

ping 192.168.1.10

Then check again:

show mac address-table

Both PC MAC addresses should remain in the table.

# 3. Add a Third PC

### Objective

To add a third PC and observe how its MAC address is learned after communication.

### Network Topology

PC1

|

|

PC2 -------- Switch -------- PC3

### PC3 Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC3 | 192.168.1.30 | 255.255.255.0 |

Connect PC3 to another switch port, for example:

PC3 → Switch Fa0/3

### Test Connectivity

From PC3:

ping 192.168.1.10

This sends traffic from PC3 to PC1.

Now check the switch:

show mac address-table

### Example Result

VLAN    MAC Address          Type       Port

----    -----------------    --------   ----

1       AAAA.BBBB.CCCC       DYNAMIC    Fa0/1

1       DDDD.EEEE.FFFF       DYNAMIC    Fa0/2

1       1111.2222.3333       DYNAMIC    Fa0/3

### Observation

The switch has now learned the MAC address of PC3 and associated it with the port where PC3 is connected.

# 4. Explain How the Switch Forwards a Frame

### Given MAC Address Table

| Port | MAC Address |
| --- | --- |
| Port 1 | 00:11:22:33:44:55 |
| Port 2 | 66:77:88:99:AA |

A frame arrives at Port 1 with destination MAC:

66:77:88:99:AA:BB

### Step-by-Step Process

Step 1: Frame Arrives

The switch receives the frame through Port 1.

Port 1 → Switch

Step 2: Switch Reads the Source MAC

The switch checks the source MAC address and can learn/update which port that MAC is using.

Step 3: Switch Checks the Destination MAC

The destination MAC is:

66:77:88:99:AA:BB

Step 4: Switch Searches Its MAC Table

The switch finds:

66:77:88:99:AA:BB → Port 2

Step 5: Switch Forwards the Frame

Because the destination MAC is already known, the switch sends the frame only through Port 2.

Port 1 → Switch → Port 2

It does not send the frame to every port because it already knows where the destination device is located.

### Final Result

The frame is successfully forwarded from Port 1 to Port 2.

# 5. MAC Learning and Frame Forwarding with Four Devices

## Network Diagram

PC1

|

|

Fa0/1

|

|

PC2 ----------- SWITCH ----------- PC3

/       \

Fa0/2      Fa0/3

|

PC3

Fa0/4

|

PC4

### Simplified Diagram

PC1

|

|

PC2 -------- SWITCH -------- PC3

|

|

PC4

### Example MAC Address Table

| Switch Port | Device | MAC Address |
| --- | --- | --- |
| Fa0/1 | PC1 | AA:AA:AA:AA:AA:01 |
| Fa0/2 | PC2 | AA:AA:AA:AA:AA:02 |
| Fa0/3 | PC3 | AA:AA:AA:AA:AA:03 |
| Fa0/4 | PC4 | AA:AA:AA:AA:AA:04 |

### Explanation

When a device sends a frame, the switch first looks at the source MAC address and remembers which port the frame came from. When another frame arrives, the switch checks its MAC table to find where the destination device is connected. If the destination MAC is known, the switch sends the frame only through the correct port instead of sending it everywhere. If the destination MAC is unknown, the switch temporarily sends the frame out through the other ports so the correct device can respond, and then the switch learns its location.