Name – parikh marmik

Introduction to network protocols theory

### 1. Wireshark Capture (www.spotify.com)

Steps:

- Open Wireshark.

- Start capturing on your active network interface.

- Open your browser and visit www.spotify.com.

- Stop the capture.

- Use the filter:

dns || tcp.port == 443 || http

Example observations:

| Packet No. | Protocol | Info |
| --- | --- | --- |
| 15 | DNS | Standard query A www.spotify.com |
| 16 | DNS | Standard query response A www.spotify.com → 35.186.xxx.xxx |
| 24 | TLS (HTTPS) | Client Hello to www.spotify.com |
| 30 | TLS (HTTPS) | Server Hello, Certificate |

Note: Modern websites like Spotify use HTTPS, so you will normally see TLS/HTTPS packets instead of plain HTTP packets.

## 2. DNS Lookup using nslookup

Command:

nslookup www.flipkart.com

Example Output:

Server:  192.168.1.1

Address: 192.168.1.1

Non-authoritative answer:

Name:    www.flipkart.com

Address: 163.53.78.128

Explanation:

The IP address (163.53.78.128 in this example) is the numerical address of the Flipkart web server. DNS translates the human-readable domain name (www.flipkart.com) into an IP address so that your computer knows where to connect.

Note: The IP address may be different on your system because websites often use multiple servers, CDNs, and load balancing.

## 3. Difference Between HTTP and HTTPS

| HTTP | HTTPS |
| --- | --- |
| Data is transmitted in plain text. | Data is encrypted using SSL/TLS. |
| Uses Port 80. | Uses Port 443. |
| Less secure; vulnerable to interception. | Secure against eavesdropping and tampering. |

Example:

I use Google, YouTube, and Amazon over HTTPS. HTTPS protects login credentials, personal information, and payment details by encrypting communication between my browser and the website.

## 4. DNS Queries and Responses in Wireshark

Filter:

dns

Suppose you search for a new website like www.python.org.

You may observe the following sequence:

| Packet | Description |
| --- | --- |
| DNS Standard query | Query for A record of www.python.org |
| DNS Standard query response | Returns IPv4 address |
| DNS Standard query | Query for AAAA record (IPv6) |
| DNS Standard query response | Returns IPv6 address (if available) |

Observation:

- The browser first sends a DNS query asking for the website's IP address.

- The DNS server replies with the corresponding IP address.

- Once the address is received, the browser begins establishing a secure HTTPS connection with the web server.

## 5. Real-World Scenario (DHCP)

Scenario:

When I connect my phone to the Wi-Fi at a coffee shop and open Instagram, the DHCP (Dynamic Host Configuration Protocol) server automatically assigns my phone an IP address, subnet mask, gateway, and DNS server. Without DHCP, I would have to configure these network settings manually before accessing the internet. DHCP makes connecting to public and home Wi-Fi quick and automatic.