# Assignment: RIP Version 2 Configuration and Packet Analysis

## 1. Set Up a Three-Router Network

### Objective

To create a network topology with three routers in Cisco Packet Tracer, connect them using serial links, and assign IP addresses to all router interfaces.

### Network Topology

PC1 ── R1 ───── R2 ───── R3 ── PC3

|        |        |

LAN 1    LAN 2    LAN 3

### IP Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 |
| R1 | S0/0/0 | 10.0.12.1 | 255.255.255.252 |
| R2 | S0/0/0 | 10.0.12.2 | 255.255.255.252 |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 |
| R2 | S0/0/1 | 10.0.23.1 | 255.255.255.252 |
| R3 | S0/0/0 | 10.0.23.2 | 255.255.255.252 |
| R3 | G0/0 | 192.168.3.1 | 255.255.255.0 |

### Router 1 Configuration

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

### Router 2 Configuration

enable

configure terminal

interface s0/0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface g0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface s0/0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

### Router 3 Configuration

enable

configure terminal

interface s0/0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface g0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

# 2. Configure RIP Version 2

### Objective

To enable RIP version 2 on all three routers, advertise directly connected networks, and verify that each router learns routes to the other subnets.

## Router 1

enable

configure terminal

router rip

version 2

no auto-summary

network 192.168.1.0

network 10.0.12.0

exit

## Router 2

enable

configure terminal

router rip

version 2

no auto-summary

network 10.0.12.0

network 192.168.2.0

network 10.0.23.0

exit

## Router 3

enable

configure terminal

router rip

version 2

no auto-summary

network 10.0.23.0

network 192.168.3.0

exit

### Verify RIP Routes

On each router, use:

show ip route

RIP-learned routes are marked with R.

Example on R1:

R    192.168.2.0/24 [120/1] via 10.0.12.2

R    192.168.3.0/24 [120/2] via 10.0.12.2

### Meaning

- R = Route learned through RIP

- 120 = RIP administrative distance

- 1 or 2 = RIP hop-count metric

- via = Next-hop router

### Expected RIP Learning

| Router | Learned Network | Hop Count |
| --- | --- | --- |
| R1 | 192.168.2.0/24 | 1 |
| R1 | 192.168.3.0/24 | 2 |
| R2 | 192.168.1.0/24 | 1 |
| R2 | 192.168.3.0/24 | 1 |
| R3 | 192.168.1.0/24 | 2 |
| R3 | 192.168.2.0/24 | 1 |

### Result

RIP version 2 successfully exchanges routing information between the three routers, and each router learns the remote networks automatically.

# 3. Change the Hop Count by Adding an Extra Router

### Objective

To observe how the RIP hop-count metric changes when an additional router is added between two existing routers.

### Original Topology

R1 ───── R2 ───── R3

From R1 to the R3 LAN:

R1 → R2 → R3

The RIP metric is approximately 2 hops.

### Modified Topology

Add Router4 between R2 and R3:

R1 ───── R2 ───── R4 ───── R3

Now the path becomes:

R1 → R2 → R4 → R3

Therefore, the hop count increases.

### Observation

Before adding Router4:

R1 → R2 → R3

Hop Count = 2

After adding Router4:

R1 → R2 → R4 → R3

Hop Count = 3

### Observation Table

| Network | Before Adding Router | After Adding Router |
| --- | --- | --- |
| R3 LAN | 2 hops | 3 hops |

### Important Note

RIP uses hop count as its routing metric. Every router crossed by a route increases the hop count by one. RIP supports a maximum metric of 15 hops; a metric of 16 means unreachable.

### Result

Adding an extra router increases the RIP hop-count metric because the packet must pass through one additional router.

# 4. Analyze RIP Update Packets in Simulation Mode

### Objective

To capture and analyze RIP update packets exchanged between routers using Cisco Packet Tracer Simulation Mode.

### Steps

- Open the three-router RIP topology in Cisco Packet Tracer.

- Click Simulation mode.

- Click Edit Filters.

- Disable unnecessary protocols.

- Select RIP as the required protocol.

- Start the simulation or use Capture/Forward.

- Observe RIP packets moving between the routers.

- Click on a RIP packet to open its details.

- Record the source router, destination router, and advertised routes.

### RIP Packet Flow

R1 ─────── RIP Update ───────> R2

R2 ─────── RIP Update ───────> R1

R2 ─────── RIP Update ───────> R3

R3 ─────── RIP Update ───────> R2

### What to Observe

When you click a RIP packet, observe:

- Source IP address

- Destination IP address

- RIP version

- Advertised network

- Hop count / metric

- Router sending the update

### Observation Table

| Source Router | Destination Router | Advertised Network | Metric |
| --- | --- | --- | --- |
| R1 | R2 | 192.168.1.0/24 | 1 |
| R2 | R1 | 192.168.2.0/24 | 1 |
| R2 | R3 | 192.168.2.0/24 | 1 |
| R3 | R2 | 192.168.3.0/24 | 1 |

Note: The exact source, destination, advertised networks, and metrics may differ according to your Packet Tracer topology.

### Result

The Simulation Mode shows that RIP routers periodically exchange routing information. These RIP updates allow routers to learn remote networks and automatically update their routing tables.