# Assignment: Static Routing Using Cisco Packet Tracer

- Set Up a Network with Two Routers

### Objective

To create a simple network with two routers connected using a serial cable and configure IP addresses on their interfaces.

### Network Topology

PC1 → Switch → Router1 → Serial Cable → Router2 → Switch → PC2

- Router1 represents Ahmedabad.

- Router2 represents Surat.

- PC1 is connected to Router1.

- PC2 is connected to Router2.

### IP Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | FastEthernet | 192.168.1.10 | 255.255.255.0 |
| Router1 | GigabitEthernet 0/0 | 192.168.1.1 | 255.255.255.0 |
| Router1 | Serial 0/0/0 | 10.0.0.1 | 255.255.255.252 |
| Router2 | Serial 0/0/0 | 10.0.0.2 | 255.255.255.252 |
| Router2 | GigabitEthernet 0/0 | 192.168.2.1 | 255.255.255.0 |
| PC2 | FastEthernet | 192.168.2.10 | 255.255.255.0 |

### Router1 Configuration

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

### Router2 Configuration

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

### PC Configuration

#### PC1

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

#### PC2

- IP Address: 192.168.2.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

- Configure Static Route on Router1

### Objective

To configure a static route on Router1 so that Router1 can reach the LAN network connected to Router2.

Router1 needs to know that the network 192.168.2.0/24 is available through Router2.

### Static Route on Router1

enable

configure terminal

ip route 192.168.2.0 255.255.255.0 10.0.0.2

exit

This command tells Router1:

"To reach the 192.168.2.0 network, send the packet to Router2 at 10.0.0.2."

### Static Route on Router2

A return route is also required on Router2.

enable

configure terminal

ip route 192.168.1.0 255.255.255.0 10.0.0.1

exit

This tells Router2:

"To reach the 192.168.1.0 network, send the packet to Router1 at 10.0.0.1."

### Testing Connectivity

From PC1, open Command Prompt and enter:

ping 192.168.2.10

If the configuration is correct, the PC should receive replies from PC2.

Example:

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

### Result

The ping test confirms that Router1 can communicate with the PC connected to Router2 through the configured static route.

# 3. Zomato-Style Delivery Network Simulation

### Objective

To simulate a delivery network where Router1 represents an Ahmedabad branch and Router2 represents a Surat branch. Static routes are configured so that devices in both cities can communicate with each other.

### Network Design

Ahmedabad LAN

192.168.1.0/24

|

PC1

|

Switch

|

Router1

192.168.1.1

|

10.0.0.1

|

Serial Link

|

10.0.0.2

|

Router2

192.168.2.1

|

Switch

|

PC2

|

Surat LAN

192.168.2.0/24

### Static Route on Ahmedabad Router

ip route 192.168.2.0 255.255.255.0 10.0.0.2

This route allows Ahmedabad devices to reach the Surat LAN.

### Static Route on Surat Router

ip route 192.168.1.0 255.255.255.0 10.0.0.1

This route allows Surat devices to reach the Ahmedabad LAN.

### Connectivity Test

From the Ahmedabad PC:

ping 192.168.2.10

From the Surat PC:

ping 192.168.1.10

If both ping tests are successful, the two LAN networks can communicate through the routers.

### Real-Life Example

This network can be compared to a Zomato-style delivery system.

- Ahmedabad = Branch 1

- Surat = Branch 2

- Router1 = Ahmedabad network router

- Router2 = Surat network router

- PCs = Branch or delivery system devices

- Serial link = Connection between the two branches

Static routes tell the routers where to send data between Ahmedabad and Surat.

# 4. Advantages and Disadvantages of Static Routing

## Advantages

### 1. Simple for Small Networks

Static routing is easy to configure when the network has only a few routers and networks.

For example, a small IRCTC branch with a limited number of networks can use static routes easily.

### 2. Low Network Overhead

Static routes do not exchange routing information between routers automatically.

Therefore, they use less bandwidth and router processing resources compared to dynamic routing protocols.

## Disadvantages

### 1. Manual Configuration is Required

The network administrator must manually configure every route.

For a large application such as Swiggy, where many branches and locations may be connected, maintaining static routes can become difficult.

### 2. Routes Do Not Automatically Change

If a network link or router fails, static routing does not automatically find another path.

For example, if the connection between two Swiggy delivery locations fails, the router will not automatically select an alternative route unless another route has been manually configured.

This can cause delays in communication and delivery updates.

# Conclusion

Static routing is a method in which network routes are manually configured by the network administrator.

In this assignment, two routers were connected using a serial cable. IP addresses were assigned to the router interfaces and PCs. Static routes were then configured to allow communication between the Ahmedabad and Surat LAN networks.

Static routing is useful for small and stable networks because it is simple and has low overhead. However, for large and changing networks, dynamic routing is generally more suitable because routes can be learned and updated automatically.