# RADIUS and TACACS+ – Assignment

## 1. RADIUS vs TACACS+

| Feature | RADIUS | TACACS+ |
| --- | --- | --- |
| Protocol type | UDP-based protocol | TCP-based protocol |
| Encryption | Encrypts the user's password; other packet information is generally not encrypted | Encrypts the entire TACACS+ packet body |
| Ports used | UDP 1812 for authentication/authorization and UDP 1813 for accounting | TCP 49 |
| Command Authorization | Limited command-level authorization | Provides detailed command-level authorization |
| AAA separation | Authentication and authorization are commonly combined | Authentication, Authorization and Accounting are separated |
| Common use | Wi-Fi, VPN and network access authentication | Network-device administrator access |

### Easy way to remember

RADIUS → Mainly user/network access

TACACS+ → Mainly administrator/device access

# 2. RADIUS Authentication Flow

Draw this flowchart in your assignment:

+-------------+

|    User     |

| Laptop/Phone|

+------+------+

|

| 1. Connect to Wi-Fi

↓

+-------------+

| Wi-Fi Access|

| Point / NAS |

+------+------+

|

| 2. Access-Request

↓

+-------------+

|   RADIUS    |

|   Server    |

+------+------+

|

| 3. Check Username/

|    Password

↓

+-------------+

| User Database|

+------+------+

|

| 4. Access-Accept

|    or Access-Reject

↓

+-------------+

| Wi-Fi Access|

| Point / NAS |

+------+------+

|

↓

User gets

Wi-Fi access

### Explanation

- The user connects to the Wi-Fi network and provides login credentials.

- The Wi-Fi Access Point/NAS sends an Access-Request to the RADIUS server.

- The RADIUS server checks the user's credentials against its authentication database.

- If the credentials are correct, the server sends Access-Accept. If they are incorrect, it sends Access-Reject.

- The access point then allows or denies the user's network access.

# 3. Why TACACS+ Is Suitable for Admin Access

### Scenario: Zomato

Imagine Zomato has many network administrators who manage routers and switches in its data centers.

An administrator logs in to a Cisco router to configure a new network interface. TACACS+ would be more suitable than RADIUS because TACACS+ can provide detailed command authorization.

For example, one administrator may be allowed to run:

show running-config

show ip interface brief

but another administrator may also be allowed to run configuration commands such as:

configure terminal

interface gigabitEthernet 0/0

TACACS+ can control and record individual administrator commands. This provides better control over network-device administration than typical RADIUS deployments.

# 4. Advantages and Disadvantages of RADIUS in Public Hotspots

### Advantages

1. Centralized Authentication
A hotspot can use one RADIUS server to authenticate many users and access points. This makes user management easier.

2. Supports Large Number of Users
RADIUS is widely used for network access such as Wi-Fi and can be integrated with user databases and access policies.

### Disadvantages

1. Security Limitation
Traditional RADIUS does not encrypt the entire packet; it specifically protects the password field, so additional security mechanisms may be needed.

2. Server Dependency
If the RADIUS server or its connectivity becomes unavailable, users may not be able to authenticate and access the hotspot.

# 5. Sample TACACS+ Configuration for Cisco Router

Below is a basic sample configuration generated for a Cisco IOS-style lab.

### TACACS+ Server

Suppose:

TACACS+ Server IP = 192.168.10.10

Shared Secret = cisco123

Router IP = 192.168.10.1

### Cisco Router Configuration

Router> enable

Router# configure terminal

Router(config)# aaa new-model

Router(config)# tacacs server TACACS-SERVER

Router(config-server-tacacs)# address ipv4 192.168.10.10

Router(config-server-tacacs)# key cisco123

Router(config-server-tacacs)# exit

Router(config)# aaa authentication login default group tacacs+ local

Router(config)# aaa authorization exec default group tacacs+ local

Router(config)# line vty 0 4

Router(config-line)# login authentication default

Router(config-line)# transport input ssh

Router(config-line)# exit

Router(config)# end

Router# write memory

### What the Important Commands Do

| Command | Purpose |
| --- | --- |
| aaa new-model | Enables AAA on the router |
| tacacs server TACACS-SERVER | Creates a TACACS+ server definition |
| address ipv4 192.168.10.10 | Specifies the TACACS+ server IP |
| key cisco123 | Sets the shared secret between router and server |
| aaa authentication login... | Defines how users are authenticated |
| aaa authorization exec... | Controls access to the router's EXEC session |
| line vty 0 4 | Configures remote VTY access |
| transport input ssh | Allows SSH remote access |

## Commands I Would Modify for a College Lab

The above configuration is only an example. In a real college Packet Tracer/lab setup, I would modify:

### 1. TACACS+ Server IP

Change:

address ipv4 192.168.10.10

to the actual IP address assigned to the TACACS+ server in the lab.

### 2. Shared Secret

Change:

key cisco123

to the shared secret configured on the TACACS+ server.

### 3. Authentication Method

For a basic lab, I would keep:

aaa authentication login default group tacacs+ local

The local option provides a local fallback account if the TACACS+ server is unavailable.

For example:

username admin privilege 15 secret class123

### 4. VTY Access

For a lab where SSH is being practiced, I would use:

transport input ssh

rather than allowing insecure Telnet.

## Important Lab Note

Some Packet Tracer versions/devices have limited TACACS+ feature support compared with real Cisco IOS devices. If a particular TACACS+ command is rejected, use the syntax supported by the router model/version in your Packet Tracer activity.

### Quick Revision

RADIUS

↓

UDP

↓

1812 / 1813

↓

Wi-Fi / Network Access

TACACS+

↓

TCP

↓

Port 49

↓

Network Administrator Access

↓

Detailed Command Authorization