# Day 5: Network Testing Practical

## Task 1: Test Connectivity Using Ping

### Objective

To check whether two devices connected to the same Wi-Fi network can communicate with each other.

### Procedure

- Connect the laptop and friend's phone to the same Wi-Fi network.

- Find the IP address of the phone.

- On the Windows laptop, open Command Prompt.

- Type the following command:

ping <phone-IP-address>

Example:

ping 192.168.1.105

- Press Enter.

- Check the response displayed by the Command Prompt.

### Result

If the devices can communicate, the result may show:

Reply from 192.168.1.105: bytes=32 time=5ms TTL=64

This means the laptop received a response from the phone.

### My Result

Phone IP Address: ______________________

Ping Result: ____________________________

Packets Sent: __________

Packets Received: __________

Packets Lost: __________

# Task 2: Traceroute to YouTube

### Objective

To find the path taken by network packets from my device to YouTube.

### Windows Procedure

- Open Command Prompt.

- Type:

tracert www.youtube.com

- Press Enter.

- Wait for the command to complete.

- Count the numbered hops shown in the output.

### Result

Destination:

Number of Hops: __________________

Time Taken: ______________________

### Explanation

tracert shows the different routers/hops through which packets travel from the local computer toward the destination.

Screenshot:
Paste the Command Prompt screenshot of the tracert www.youtube.com output here.

The exact number of hops can change depending on your ISP, routing path, destination server, and network conditions.

# Task 3: Ping After Disconnecting the Device

### Procedure

- First connect the phone to the same Wi-Fi network.

- Find its IP address.

- Ping the phone from the laptop and confirm that it responds.

- Disconnect the phone from Wi-Fi.

- Run the ping command again using the same IP address.

ping <phone-IP-address>

### Expected Result

The ping will normally show a failure such as:

Request timed out.

or

Destination host unreachable.

The exact message depends on the operating system and network.

### Explanation

After disconnecting the phone from Wi-Fi, it is no longer reachable at its previous network address. Therefore, the laptop may not receive a reply to the ping request.

Observed Error Message: __________________________

# Task 4: List Devices Using arp -a

### Procedure

- Connect the laptop to your home Wi-Fi.

- Open Command Prompt.

- First communicate with some devices on the network if needed, because the ARP table only contains entries the computer has recently learned.

- Type:

arp -a

- Press Enter.

- Record the IP addresses and MAC addresses shown.

### IP Address Table

| No. | IP Address | MAC Address | Device/Person |
| --- | --- | --- | --- |
| 1 | __________________ | __________________ | Laptop – Me |
| 2 | __________________ | __________________ | Phone – Friend |
| 3 | __________________ | __________________ | __________________ |
| 4 | __________________ | __________________ | __________________ |
| 5 | __________________ | __________________ | __________________ |

### Example

| No. | IP Address | MAC Address | Device |
| --- | --- | --- | --- |
| 1 | 192.168.1.10 | XX-XX-XX-XX-XX-XX | My Laptop |
| 2 | 192.168.1.11 | XX-XX-XX-XX-XX-XX | Smartphone |
| 3 | 192.168.1.12 | XX-XX-XX-XX-XX-XX | Smart TV |

.

# Task 5: One-Page Network Testing Report

## Report: Setting Up and Testing a Basic Wi-Fi Network

### Introduction

The purpose of this practical was to set up a basic Wi-Fi network and test connectivity between devices. I connected my laptop and a smartphone to the same Wi-Fi network and used network commands such as ping, tracert, and arp -a to understand how the devices communicate.

### Network Setup

First, I connected my laptop and smartphone to the same home Wi-Fi network. I checked the IP address of the devices and made sure they were connected to the same local network. The laptop was used as the main device for performing the network tests.

### Ping Test

I used the ping command from Command Prompt to test connectivity between the laptop and smartphone. The command used was:

ping <phone-IP-address>

The laptop received __________ replies from the smartphone, with __________% packet loss. This showed that the device was reachable / not reachable from the laptop.

### Traceroute Test

Next, I used the following command to check the route to YouTube:

tracert www.youtube.com

The command displayed multiple network hops between my device and the destination. The total number of observed hops was __________.

Traceroute is useful for understanding the path taken by packets and can help identify where delays or connectivity problems may occur.

### Disconnection Test

After disconnecting the smartphone from Wi-Fi, I tried the ping command again. The laptop did not receive a normal reply and displayed ________________________. This demonstrated that a disconnected device is normally no longer reachable through the local Wi-Fi network.