# Day 3: BIOS and POST Troubleshooting

## Task 1: Common BIOS or POST Error Messages

| No. | BIOS/POST Error Message | Usually Indicates |
| --- | --- | --- |
| 1 | CMOS Checksum Error – Defaults Loaded | CMOS battery may be weak/dead or BIOS settings may be corrupted. |
| 2 | Keyboard Error or No Keyboard Present | Keyboard may be disconnected, faulty, or not detected by the motherboard. |
| 3 | CMOS Battery Low | The CMOS/RTC battery is weak and may need replacement. |
| 4 | No Bootable Device Found | The HDD/SSD may not be detected or the boot device/order may be incorrect. |
| 5 | CPU Fan Error | CPU fan may not be connected, may be faulty, or may not be spinning properly. |

## Task 2: Three Beeps on an AMI BIOS

Scenario:
The PC gives 3 beeps repeatedly during startup and there is no display.

Meaning:
For a traditional AMI BIOS, 3 short beeps typically indicate a base/first 64 KB RAM memory error. However, beep codes can vary by BIOS version and motherboard, so the motherboard's documentation should be checked.

### Troubleshooting Steps:

Step 1 – Reseat the RAM

- Turn OFF the computer and disconnect the power.

- Remove the RAM module carefully.

- Clean the contacts if necessary and reinstall the RAM firmly.

- Start the PC again and check whether the display returns.

Step 2 – Test the RAM

- If there are multiple RAM sticks, test them one at a time.

- Try another compatible RAM slot.

- If possible, test with a known-good compatible RAM module.

## Task 3: BIOS Setup Utility

### Procedure:

- Turn ON or restart the computer.

- During startup, repeatedly press the BIOS setup key.

- Common keys are Delete, F2, F10, or Esc, depending on the manufacturer.

- Open the main BIOS/UEFI information screen.

- Identify the motherboard manufacturer and BIOS/UEFI version.

- Take a screenshot of the main BIOS screen.

.

## Task 4: CMOS Checksum Error – Defaults Loaded

### Meaning:

“CMOS checksum error – Defaults loaded” means that the BIOS/UEFI detected that the saved CMOS configuration data is invalid or does not match its expected checksum.

A common cause is a weak or dead CMOS/RTC battery. It can also happen after BIOS settings become corrupted or are reset.

### How to Resolve It:

- Enter the BIOS/UEFI setup.

- Check and correct the date and time.

- Check important settings such as boot order and storage configuration.

- Save the correct settings and restart the PC.

- If the error appears again, turn off the PC and disconnect power.

- Replace the CMOS/RTC battery with the correct type, commonly a CR2032 on desktop motherboards.

- Start the PC and configure the BIOS settings again.

- Save the settings and restart.