# Assignment: VLAN Trunking, 802.1Q and ISL

## 1. Configure Two Switches with a Trunk Link

### Objective

To connect two Cisco switches and configure FastEthernet0/1 on both switches as trunk ports using IEEE 802.1Q.

### Network Topology

Switch1 ================= Switch2

Trunk Link

Fa0/1 ↔ Fa0/1

Connect Switch1 and Switch2 using a suitable Ethernet cable. In Packet Tracer, the trunk can be configured manually, so the exact cable type may depend on the switch model.

### Switch1 Configuration

enable

configure terminal

interface fastEthernet 0/1

switchport mode trunk

no shutdown

exit

### Switch2 Configuration

enable

configure terminal

interface fastEthernet 0/1

switchport mode trunk

no shutdown

exit

### Verify Trunk

Use:

show interfaces trunk

The output should show FastEthernet0/1 as a trunk port.

### 802.1Q

802.1Q is the standard VLAN trunking method used to carry traffic from multiple VLANs across one physical trunk link.

# 2. Create VLANs and Assign Access Ports

### Objective

To create VLAN 10, VLAN 20, and VLAN 30 on both switches and assign ports on Switch1 to different VLANs.

## Create VLANs on Switch1

enable

configure terminal

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

## Create VLANs on Switch2

enable

configure terminal

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

## Assign Switch1 Ports

### Fa0/2 → VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

### Fa0/3 → VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

### Fa0/4 → VLAN 30

interface fastEthernet 0/4

switchport mode access

switchport access vlan 30

exit

### Verify VLAN Configuration

show vlan brief

### Expected Configuration

| Switch | Port | VLAN | Mode |
| --- | --- | --- | --- |
| Switch1 | Fa0/2 | VLAN 10 | Access |
| Switch1 | Fa0/3 | VLAN 20 | Access |
| Switch1 | Fa0/4 | VLAN 30 | Access |
| Switch1 | Fa0/1 | All VLANs | Trunk |

# 3. Connect PCs and Test VLAN Communication

### Objective

To verify that devices in different VLANs cannot communicate directly, while devices in the same VLAN can communicate when they are connected through the same VLAN.

### Network Topology

PC1 ── Fa0/2 ──┐

│ VLAN 10

│

PC2 ── Fa0/3 ──┤ Switch1 ===== Switch2

│    Trunk

PC3 ── Fa0/4 ──┘

### PC IP Addresses

| PC | VLAN | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | 10 | 192.168.10.10 | 255.255.255.0 |
| PC2 | 20 | 192.168.20.10 | 255.255.255.0 |
| PC3 | 30 | 192.168.30.10 | 255.255.255.0 |

### Test 1: PC1 to PC2

From PC1:

ping 192.168.20.10

Expected result:

Request timed out.

PC1 and PC2 are in different VLANs, so they cannot communicate directly without inter-VLAN routing.

### Test 2: PC1 to PC3

ping 192.168.30.10

Expected result:

Request timed out.

Again, they are in different VLANs.

### Same VLAN Communication

To test communication between PCs in the same VLAN, connect another PC to a port assigned to VLAN 10.

For example:

PC1 → VLAN 10

PC4 → VLAN 10

If PC4 has IP:

192.168.10.20

then from PC1:

ping 192.168.10.20

The ping should be successful.

### Observation

| Source | Destination | VLAN | Expected Result |
| --- | --- | --- | --- |
| PC1 | PC2 | 10 → 20 | Fail |
| PC1 | PC3 | 10 → 30 | Fail |
| PC1 | PC4 | 10 → 10 | Success |

### Explanation

VLANs divide a switch into separate logical networks. Devices in different VLANs are isolated at Layer 2. To allow communication between different VLANs, a router or Layer 3 switch is required.

# 4. Change Trunk Encapsulation from 802.1Q to ISL

### Objective

To test whether VLAN traffic continues to pass when the trunk encapsulation is changed from 802.1Q to ISL.

### Important Note

ISL is a Cisco proprietary trunking protocol and is supported only on certain older Cisco switch models. Many Packet Tracer switches, especially the Cisco 2960, support 802.1Q only and do not support ISL.

Therefore, the following command may not work:

switchport trunk encapsulation isl

### If the Switch Supports ISL

On Switch1:

enable

configure terminal

interface fastEthernet 0/1

switchport trunk encapsulation isl

switchport mode trunk

exit

On Switch2:

enable

configure terminal

interface fastEthernet 0/1

switchport trunk encapsulation isl

switchport mode trunk

exit

Then verify:

show interfaces trunk

### If the Command Is Not Supported

The switch may display an error because the switch supports only 802.1Q.

In that case, keep the trunk as:

interface fastEthernet 0/1

switchport mode trunk

and use 802.1Q.

### Result

If both switches support ISL and the same encapsulation is configured on both sides, VLAN traffic can pass using ISL.

If the Packet Tracer switch does not support ISL, VLAN traffic cannot be tested using ISL on that device. This is a limitation of the switch model, not a configuration mistake.

# 5. Accidentally Configure Switch2 Fa0/5 as a Trunk

### Objective

To observe what happens when a port intended for a PC in VLAN 20 is accidentally configured as a trunk port.

### Correct Configuration

If PC4 should belong to VLAN 20, Fa0/5 should normally be configured as an access port:

interface fastEthernet 0/5

switchport mode access

switchport access vlan 20

exit

### Accidental Configuration

Suppose Fa0/5 is incorrectly configured as a trunk:

interface fastEthernet 0/5

switchport mode trunk

exit

Now the port is operating as a trunk instead of a normal access port.

### Impact on the PC

A normal PC sends untagged Ethernet frames. A trunk port is designed to carry traffic for multiple VLANs and normally expects VLAN-tagged traffic.

Therefore, the PC connected to Fa0/5 may not communicate as expected with devices in VLAN 20.

For example:

PC4

|

Fa0/5

|

Switch2

If PC4 is supposed to be in VLAN 20, the correct configuration is:

Fa0/5 → Access → VLAN 20

not:

Fa0/5 → Trunk

### Correct the Configuration

enable

configure terminal

interface fastEthernet 0/5

switchport mode access

switchport access vlan 20

exit

Then verify:

show vlan brief

and test connectivity:

ping <VLAN 20 PC IP>

### Observation

| Configuration | Expected Behavior |
| --- | --- |
| Fa0/5 Access VLAN 20 | PC works normally in VLAN 20 |
| Fa0/5 Trunk | PC may have connectivity problems because the port is not configured as a normal host/access port |