# Day 4: Network Cable Types and Tools

## Task 1: Ethernet Cable vs Fiber Optic Cable

| Feature | Ethernet Cable | Fiber Optic Cable |
| --- | --- | --- |
| Transmission | Uses electrical signals to carry data. | Uses light signals to carry data. |
| Distance | Suitable for short and medium distances, commonly up to 100 m for standard copper Ethernet links. | Suitable for much longer distances. |
| Cost | Generally cheaper and easier to install. | Usually more expensive and requires specialized tools. |
| Interference | Can be affected by electromagnetic interference. | Resistant to electromagnetic interference. |

### Real-Life Examples

Ethernet: Used to connect a home router/switch to a PC, gaming console, or smart TV.

Fiber Optic: Used for high-speed backbone connections in data centers, ISPs, and college campuses.

# Task 2: Straight-Through Ethernet Cable – T568B

A straight-through cable uses the same wiring standard on both ends. For this assignment, we use T568B on both ends.

### T568B Pinout

RJ45 Connector – T568B

┌─────────────────────────┐

Pin 1  │ White/Orange            │

Pin 2  │ Orange                  │

Pin 3  │ White/Green             │

Pin 4  │ Blue                    │

Pin 5  │ White/Blue              │

Pin 6  │ Green                   │

Pin 7  │ White/Brown             │

Pin 8  │ Brown                   │

└─────────────────────────┘

END A              END B

T568B               T568B

│                    │

│ Same order         │

└────────────────────┘

### T568B Color Order

| Pin | Wire Color |
| --- | --- |
| 1 | White/Orange |
| 2 | Orange |
| 3 | White/Green |
| 4 | Blue |
| 5 | White/Blue |
| 6 | Green |
| 7 | White/Brown |
| 8 | Brown |

# Task 3: How to Attach an RJ45 Connector to a CAT6 Cable

### Tools Required:

- CAT6 Ethernet cable

- RJ45 connector

- RJ45 crimping tool

- Cable stripper/cutter

- Cable tester

### Step-by-Step Procedure

Step 1 – Cut the Cable

Cut the CAT6 cable to the required length using a cable cutter.

Step 2 – Remove the Outer Jacket

Use the stripper to carefully remove a small section of the outer jacket. Do not damage the internal wires.

Step 3 – Untwist the Wires

Separate the four twisted pairs and straighten the individual wires.

Step 4 – Arrange the Wires

Arrange the wires according to the T568B standard:

White/Orange → Orange → White/Green → Blue → White/Blue → Green → White/Brown → Brown

Step 5 – Trim the Wires

Hold the wires tightly in the correct order and trim their ends evenly.

Step 6 – Insert into RJ45 Connector

Insert all eight wires into the RJ45 connector. Make sure every wire reaches the correct pin and the cable jacket enters the connector.

Step 7 – Crimp the Connector

Place the RJ45 connector into the crimping tool and squeeze firmly to lock the contacts into the wires.

Step 8 – Repeat for the Other End

For a straight-through cable, use the same T568B order on the other end.

Step 9 – Test the Cable

Connect both ends to an Ethernet cable tester and check that pins 1–8 are correctly connected.

# Task 4: Gaming Café – Cable and Tool Selection

For a gaming café with 10 PCs, I would choose Ethernet CAT6 cable because it provides good speed and low latency at a reasonable cost for a typical LAN gaming setup. Ethernet copper cabling is easier and cheaper to install for short distances inside a café. I would use an RJ45 crimper to attach RJ45 connectors to the CAT6 cables. Fiber optic would be more useful for longer-distance or backbone connections where very high bandwidth and resistance to interference are important.