# Assignment: Router-on-a-Stick and Inter-VLAN Routing

## 1. Create Three VLANs for Zomato, Flipkart and Spotify Users

### Objective

To create three VLANs representing different types of users:

- Foodies → Zomato users

- Shoppers → Flipkart users

- Streamers → Spotify users

At least one PC is assigned to each VLAN.

### VLAN Table

| VLAN ID | VLAN Name | Application | PC |
| --- | --- | --- | --- |
| 10 | Foodies | Zomato | PC1 |
| 20 | Shoppers | Flipkart | PC2 |
| 30 | Streamers | Spotify | PC3 |

### Network Topology

Router

|

G0/0 Trunk

|

Switch

/      |      \

/        |        \

PC1       PC2       PC3

VLAN 10    VLAN 20    VLAN 30

Foodies    Shoppers   Streamers

Zomato     Flipkart   Spotify

# 2. Configure Router-on-a-Stick

### Objective

To allow PCs in different VLANs to communicate using router sub-interfaces.

Router-on-a-stick uses one physical router interface with multiple logical sub-interfaces.

The router interface will be:

GigabitEthernet0/0

The sub-interfaces will be:

G0/0.10 → VLAN 10

G0/0.20 → VLAN 20

G0/0.30 → VLAN 30

## Step 1: Create VLANs on the Switch

enable

configure terminal

vlan 10

name Foodies

exit

vlan 20

name Shoppers

exit

vlan 30

name Streamers

exit

## Step 2: Assign PC Ports to VLANs

### PC1 → VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

### PC2 → VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

### PC3 → VLAN 30

interface fastEthernet 0/4

switchport mode access

switchport access vlan 30

exit

# Step 3: Configure the Switch-to-Router Port as Trunk

Suppose the router is connected to Switch port Fa0/1.

interface fastEthernet 0/1

switchport mode trunk

exit

Verify:

show interfaces trunk

Fa0/1 should appear as a trunk port.

# Step 4: Configure Router Sub-Interfaces

First enable the physical interface:

enable

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

## VLAN 10 – Foodies

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

## VLAN 20 – Shoppers

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

## VLAN 30 – Streamers

interface gigabitEthernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

### Router Configuration Summary

| Sub-interface | VLAN | IP Address | Purpose |
| --- | --- | --- | --- |
| G0/0.10 | 10 | 192.168.10.1 | Foodies |
| G0/0.20 | 20 | 192.168.20.1 | Shoppers |
| G0/0.30 | 30 | 192.168.30.1 | Streamers |

# 3. Configure IP Addresses on PCs

## PC1 – Foodies / Zomato

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

## PC2 – Shoppers / Flipkart

IP Address: 192.168.20.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.20.1

## PC3 – Streamers / Spotify

IP Address: 192.168.30.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.30.1

### IP Address Table

| PC | VLAN | Application | IP Address | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | 10 | Zomato | 192.168.10.10 | 192.168.10.1 |
| PC2 | 20 | Flipkart | 192.168.20.10 | 192.168.20.1 |
| PC3 | 30 | Spotify | 192.168.30.10 | 192.168.30.1 |

# Test Inter-VLAN Connectivity

From PC1:

ping 192.168.20.10

ping 192.168.30.10

From PC2:

ping 192.168.10.10

ping 192.168.30.10

From PC3:

ping 192.168.10.10

ping 192.168.20.10

All pings should be successful if the configuration is correct.

### Why Does This Work?

For example, when PC2 wants to communicate with PC3:

PC2

↓

VLAN 20

↓

Switch

↓

Trunk

↓

Router G0/0.20

↓

Routing

↓

Router G0/0.30

↓

Trunk

↓

Switch

↓

PC3

The router performs the routing between VLAN 20 and VLAN 30.

# 4. Troubleshoot: Shoppers Cannot Ping Streamers

### Problem

Suppose:

PC2 (VLAN 20)

↓

Cannot ping

↓

PC3 (VLAN 30)

We need to find the configuration problem.

## Step 1: Check VLAN Assignment

On the switch:

show vlan brief

Verify:

Fa0/3 → VLAN 20

Fa0/4 → VLAN 30

If PC3 is accidentally assigned to VLAN 20, correct it:

interface fastEthernet 0/4

switchport mode access

switchport access vlan 30

exit

## Step 2: Check the Trunk

Use:

show interfaces trunk

The switch port connected to the router must be a trunk.

If it is not:

interface fastEthernet 0/1

switchport mode trunk

exit

## Step 3: Check Router Sub-Interface for VLAN 20

show ip interface brief

Verify that:

G0/0.20 → 192.168.20.1

is up.

Check configuration:

show running-config

The configuration should contain:

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

## Step 4: Check Router Sub-Interface for VLAN 30

It should contain:

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

If the VLAN ID is wrong, correct it:

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

## Step 5: Check PC Default Gateways

PC2:

Default Gateway: 192.168.20.1

PC3:

Default Gateway: 192.168.30.1

If the gateways are wrong, inter-VLAN communication will fail.

## Final Test

From PC2:

ping 192.168.30.10

Expected:

Reply from 192.168.30.10

Reply from 192.168.30.10

### Result

The connectivity problem is fixed after correcting the VLAN assignment, trunk configuration, router sub-interface, or default gateway.

# 5. Router-on-a-Stick vs Layer 3 Switch

| Point | Router-on-a-Stick | Layer 3 Switch |
| --- | --- | --- |
| Advantage 1 | Less expensive if a router is already available. | Faster inter-VLAN routing because routing is performed directly by the Layer 3 switch. |
| Advantage 2 | Easy to understand and configure for small networks. | Can handle many VLANs and high traffic more efficiently. |
| Disadvantage 1 | One physical link carries traffic for all VLANs, which can become a bottleneck. | Layer 3 switches are generally more expensive than basic Layer 2 switches. |
| Disadvantage 2 | Router processing can limit performance when there is heavy inter-VLAN traffic. | Configuration can be more advanced compared with a simple router-on-a-stick setup. |

### Simple Comparison

Router-on-a-Stick:

VLAN 10 ─┐

VLAN 20 ─┼── Switch ── Trunk ── Router

VLAN 30 ─┘                  ↑

Routing

Layer 3 Switch:

VLAN 10 ─┐

VLAN 20 ─┼── Layer 3 Switch

VLAN 30 ─┘       ↑

Routing

### Which is Better?

For a small network or learning environment, router-on-a-stick is simple and cost-effective.

For a large college, office, or enterprise network with heavy traffic, a Layer 3 switch is usually more suitable because it can perform inter-VLAN routing faster and more efficiently.