# Name – parikh marmik

# Computer Networks Assignment - Network Cables

## Cat6 Ethernet Cable (T568B)

RJ-45 Pins (T568B)
Pin1 White/Orange
Pin2 Orange
Pin3 White/Green
Pin4 Blue
Pin5 White/Blue
Pin6 Green
Pin7 White/Brown
Pin8 Brown

Diagram:
[RJ45]
1 WO |2 O |3 WG |4 B |5 WB |6 G |7 WBr |8 Br

## 2. Types of Network Cables

| Ans- Cable | Example | Reason |
| --- | --- | --- |
| Cat5e | WiFi Router | Suitable for home/office Gigabit LAN |
| Cat6 | Gaming PC / Zomato POS | Higher speed and lower interference |
| Fiber Optic | IPL Streaming Server | Very high speed over long distances |

## 3. RJ-45 Crimping Steps

Ans-

Cut cable.

Strip outer jacket.

Arrange wires in T568B order.

Trim evenly.

Insert into RJ45.

Crimp using tool.

Test cable.

Common mistakes:
• Wrong wire order.
• Wires not fully inserted before crimping.

## 4. Gaming Zone Scenario

Ans- Use Cat6 Ethernet cables with RJ-45 connectors. Cat6 provides high-speed, low-latency, reliable connections for gaming consoles, PCs, switches, and streaming displays.

## 5. Network Cable Photo

Ans- Attach a photo of an Ethernet cable (RJ-45) from your home/college or online. Description: This is a Cat6 Ethernet cable with an RJ-45 connector connecting a WiFi router to a computer or network switch.