# SD-WAN – Assignment

## 1. Traditional WAN vs SD-WAN

| Feature | Traditional WAN | SD-WAN |
| --- | --- | --- |
| 1. Data Routing | Traffic usually follows manually configured/static paths | Traffic can automatically choose the best available path |
| 2. Management | Each router/device is often configured separately | Centralized management through an SD-WAN controller |
| 3. WAN Links | Commonly depends heavily on MPLS/private circuits | Can use MPLS, broadband, 4G/5G, and other links together |
| 4. Security | Security is often provided by separate firewalls/security devices | Security features can be integrated into the SD-WAN solution |
| 5. Cloud Access | Traffic may need to travel through the main data center before reaching cloud services | Can provide more direct and optimized access to cloud applications |
| 6. Cost | Private WAN links such as MPLS can be expensive | Can reduce cost by using lower-cost broadband alongside MPLS |
| 7. Traffic Control | Less flexible and often manually configured | Applications can be prioritized automatically according to policies |

### Simple Difference

Traditional WAN:
Branch → Fixed WAN path → Data Center → Application

SD-WAN:
Branch → Best available path → Application

# 2. SD-WAN Network Diagram

You can draw the following diagram on paper or in draw.io:

+----------------------+

|   SD-WAN Controller  |

|   Central Management |

+----------+-----------+

|

SD-WAN Control

|

+---------------------+---------------------+

|                     |                     |

↓                     ↓                     ↓

+-------------+       +-------------+       +-------------+

| Ahmedabad   |       |   Mumbai    |       |  Bangalore  |

| SD-WAN Edge |       | SD-WAN Edge |       | SD-WAN Edge |

+------+------+       +------+------+       +------+------+

|                     |                     |

+---+---+             +---+---+             +---+---+

|       |             |       |             |       |

MPLS    Broadband      MPLS    Broadband      MPLS    4G/5G

|       |             |       |             |       |

+---+---+             +---+---+             +---+---+

|                     |                     |

+----------+----------+----------+----------+

|

↓

+-------------------+

|   Main Data Center|

+-------------------+

|

↓

+-------------+

| Cloud Apps  |

| Office 365  |

|    AWS      |

+-------------+

### Labels to mention

- SD-WAN Controller: Centrally manages policies and SD-WAN devices.

- SD-WAN Edge: Device at each branch that forwards traffic and selects WAN paths.

- MPLS: Private/managed WAN connection.

- Broadband: Internet-based WAN connection.

- 4G/5G: Wireless WAN connection.

- Data Center: Hosts company applications and services.

- Cloud: Services such as Office 365 and AWS.

# 3. SD-WAN Example – Zomato

Imagine Zomato has offices in Ahmedabad, Mumbai, and Bangalore and employees need to access applications and cloud services from all three cities.

With traditional WAN, traffic may have to follow a fixed route even when that connection becomes slow. With SD-WAN, the system can monitor available links and select a better path. For example, if the MPLS connection in Mumbai becomes congested, SD-WAN can use broadband for suitable application traffic. This can improve speed and reliability, while SD-WAN security policies help protect company traffic.

### Simple Example

Mumbai Office

|

MPLS is slow

↓

SD-WAN detects problem

↓

Uses Broadband

↓

Better connection

↓

Zomato Cloud Application

Result: Employees get a more reliable connection to applications even when one WAN link has problems.

# 4. Three Key Benefits of SD-WAN

## 1. Better Reliability

SD-WAN can use multiple WAN connections such as MPLS + broadband + 4G/5G.

Real-world example:
If the MPLS link at the Ahmedabad office goes down, SD-WAN can move traffic to the broadband connection. Employees can continue accessing company applications instead of completely losing connectivity.

## 2. Better Performance

SD-WAN can identify application traffic and select an appropriate WAN path based on network conditions.

Real-world example:
Employees in the Bangalore office are attending an important video meeting. If one link has high latency or packet loss, SD-WAN can use a better available link to improve the meeting experience.

## 3. Centralized Management and Security

An administrator can create network policies centrally instead of configuring every branch router individually. Security policies can also be applied consistently across Ahmedabad, Mumbai, and Bangalore.

Real-world example:
If the company wants to restrict access to a particular application, the administrator can create a policy centrally and apply it to all three offices.