Name – parikh marmik

VLAN Configuration Assignment

## 1. Network Diagram with VLANs

### Network Diagram

+----------------+
                     |     Switch     |
                     +----------------+
                    /        |         \
                   /         |          \
                  /          |           \
         +-----------+ +--------------+ +-----------+
         | Laptop    | | Friend Phone | | Smart TV  |
         +-----------+ +--------------+ +-----------+
          VLAN 20        VLAN 20         VLAN 10
       (Personal)      (Personal)     (Streaming)

### VLAN Assignment Table

| Device | VLAN ID | Purpose |
| --- | --- | --- |
| Laptop | VLAN 20 | Personal Device |
| Friend's Phone | VLAN 20 | Personal Device |
| Smart TV | VLAN 10 | Streaming |

# 2. Configure Two VLANs in Cisco Packet Tracer

## Objective

- Create VLAN 10 and VLAN 20.

- Assign three PCs to VLAN 10.

- Assign two PCs to VLAN 20.

- Verify that devices in different VLANs cannot communicate.

### Network Setup

| PC | Port | VLAN |
| --- | --- | --- |
| PC1 | Fa0/1 | VLAN 10 |
| PC2 | Fa0/2 | VLAN 10 |
| PC3 | Fa0/3 | VLAN 10 |
| PC4 | Fa0/4 | VLAN 20 |
| PC5 | Fa0/5 | VLAN 20 |

### Switch Configuration

enable
configure terminal

vlan 10
name STREAMING
exit

vlan 20
name PERSONAL
exit

interface range fa0/1-3
switchport mode access
switchport access vlan 10
exit

interface range fa0/4-5
switchport mode access
switchport access vlan 20
exit

end
write memory

### Verify VLANs

show vlan brief

### Expected Output

| VLAN | Ports |
| --- | --- |
| VLAN 10 | Fa0/1, Fa0/2, Fa0/3 |
| VLAN 20 | Fa0/4, Fa0/5 |

### Ping Test

| Source | Destination | Result |
| --- | --- | --- |
| PC1 → PC2 | Same VLAN | Successful |
| PC1 → PC4 | Different VLAN | Failed |
| PC3 → PC5 | Different VLAN | Failed |

Explanation:
Devices in the same VLAN can communicate successfully. Devices in different VLANs cannot communicate because a Layer 2 switch does not perform routing between VLANs.

# 3. Move a PC from VLAN 10 to VLAN 20

## Before Configuration

show vlan brief

VLAN 10
Fa0/1
Fa0/2
Fa0/3

VLAN 20
Fa0/4
Fa0/5

PC3 is connected to Fa0/3 and belongs to VLAN 10.

### Move PC3 to VLAN 20

enable
configure terminal

interface fa0/3
switchport mode access
switchport access vlan 20

end
write memory

## After Configuration

show vlan brief

VLAN 10
Fa0/1
Fa0/2

VLAN 20
Fa0/3
Fa0/4
Fa0/5

### Connectivity Test

| Test | Result |
| --- | --- |
| PC3 → PC1 | Failed |
| PC3 → PC2 | Failed |
| PC3 → PC4 | Successful |
| PC3 → PC5 | Successful |

Conclusion:
After moving PC3 to VLAN 20, it can communicate only with devices in VLAN 20 and no longer with devices in VLAN 10.

# 4. Automate Assigning 10 Ports to VLAN 30

### Configuration

enable
configure terminal

vlan 30
name OFFICE
exit

interface range fa0/1-10
switchport mode access
switchport access vlan 30

end
write memory

### Verification

show vlan brief

### Expected Output

| VLAN | Assigned Ports |
| --- | --- |
| VLAN 30 | Fa0/1 to Fa0/10 |