# Day 5: Case Study Practical

## Task 1: Laptop Will Not Power ON

Scenario: A user reports that the laptop will not power on even when it is plugged in.

| Step | Troubleshooting Action | What I Am Checking |
| --- | --- | --- |
| 1. Check the power socket | Connect another device to the same socket. | Checks whether the wall socket is providing power. |
| 2. Inspect the charger | Check the charger, cable, connector, and adapter for physical damage. | Checks whether the charger is damaged or disconnected. |
| 3. Test the charger output | Use a multimeter to measure the DC output of the charger and compare it with the rating printed on the adapter. | Checks whether the charger is providing the correct voltage. |
| 4. Check the charging port | Inspect the laptop's charging port for looseness, dirt, or physical damage. | Checks whether power can properly enter the laptop. |
| 5. Check the battery and internal connections | If safely accessible, inspect the battery connector and look for signs of damage. | Checks whether the battery or internal power connection is causing the no-power condition. |

# Task 2: Monitor Shows a Blank Screen

Helpdesk Ticket: “My monitor is showing a blank screen, but the CPU seems to be running.”

### Troubleshooting Flow

Step 1 – Check the monitor power

Make sure the monitor is switched ON and the power LED is working.

Step 2 – Check the display cable

Check the HDMI, DisplayPort, or VGA cable at both ends. Reconnect it firmly.

Step 3 – Check the correct input source

Use the monitor's input/source button and select the correct HDMI/DisplayPort/VGA input.

Step 4 – Check the display output port

If the PC has a dedicated GPU, make sure the display cable is connected to the GPU output, not the motherboard output, when the system is configured to use the dedicated GPU.

Step 5 – Test another cable or monitor

Use a known-good cable or another working monitor to determine whether the problem is with the monitor or cable.

Step 6 – Check the GPU

Make sure the graphics card is properly seated and its required power connectors are connected.

Step 7 – Check basic display settings

If the computer can be accessed remotely or another display works, check the display settings and make sure the monitor is detected.

Step 8 – Restart and confirm

Restart the PC and check whether the display returns.

# Task 3: Wireless Mouse Is Unresponsive

Scenario: The keyboard works, but the wireless mouse does not respond.

### Troubleshooting Steps

Step 1 – Check the mouse

- Check whether the mouse is switched ON.

- Replace the battery or charge the mouse.

- Check whether the optical sensor is clean.

Step 2 – Check the USB receiver

- Remove the wireless USB receiver.

- Reconnect it to the computer.

- Try another USB port.

Step 3 – Test the USB port

Connect another working USB device, such as a USB flash drive, to the same port.

- If the other device works → the USB port is probably working.

- If it does not work → the USB port or system may have a problem.

Step 4 – Test the mouse on another computer

Connect the mouse/receiver to another computer.

- If it works → the original system may have a settings/USB issue.

- If it does not work → the mouse, battery, or receiver may be faulty.

Step 5 – Check system settings

Check whether the mouse is detected by the operating system and whether there is a driver or device-related problem.

# Task 4: Gaming PC Overheating and Shutting Down

### Possible Cause 1: Dust Blocking Airflow

Problem: Dust can collect on the CPU heatsink, GPU heatsink, and case filters.

How to Check:

- Turn off and unplug the PC.

- Open the case.

- Inspect the fans, heatsinks, and air filters for excessive dust.

- Clean the dust carefully using appropriate cleaning equipment.

### Possible Cause 2: Faulty or Slow Fan

Problem: A CPU or case fan may not be spinning properly.

How to Check:

- Turn off the PC and inspect the fan.

- Check whether the fan is physically blocked.

- Make sure the fan cable is properly connected.

- Start the PC and verify that the fan spins normally.

### Possible Cause 3: Old or Poor Thermal Paste

Problem: Old or dried thermal paste can reduce heat transfer between the CPU and heatsink.

How to Check:

- If the CPU is overheating despite a clean heatsink and working fan, inspect the thermal paste.

- If necessary, remove the heatsink, clean the old thermal paste, and apply new thermal paste correctly.

- Reinstall the heatsink firmly.