# Day 4: Component-Specific Troubleshooting

## Task 1: RAM Reseating Procedure

Objective: To reseat the RAM module and check whether the computer recognizes it properly.

### Steps:

- Shut down the computer completely.

- Disconnect the power cable from the PC.

- Press the power button for a few seconds to discharge remaining power.

- Open the computer case carefully.

- Locate the RAM slots on the motherboard.

- Push the locking clips on both sides of the RAM slot outward.

- Carefully remove the RAM module by holding its edges.

- Check the RAM module and slot for dust or physical damage.

- Align the notch on the RAM module with the notch in the slot.

- Press the RAM module firmly and evenly until the locking clips click into place.

- Close the computer case and reconnect the power cable.

- Turn ON the computer and check whether it boots normally.

# Task 2: GPU Troubleshooting Checklist

### Faulty GPU Troubleshooting Checklist

- 1. Identify the symptom – Check whether there is no display, screen flickering, artifacts, or random crashes.

- 2. Check the display cable – Make sure the HDMI/DisplayPort cable is connected properly to the GPU and monitor.

- 3. Check GPU power connections – Make sure all required PCIe power connectors are firmly connected.

- 4. Reseat the GPU – Turn off the PC, disconnect power, remove the GPU, and reinstall it firmly in the PCIe slot.

- 5. Check the monitor/input source – Confirm that the monitor is working and the correct input source is selected.

- 6. Check drivers – Verify that the correct GPU driver is installed and functioning properly.

- 7. Test the GPU – Run a suitable graphics test/game and check for artifacts, crashes, or overheating.

- 8. Confirm the fix – Restart the PC and verify that the display works normally without crashes or artifacts.

# Task 3: Storage Issue Simulation

Objective: To observe what happens when an SSD/HDD is disconnected and then reconnected.

### Steps:

- Shut down the test computer.

- Disconnect the power supply.

- For a SATA drive, disconnect the SATA data cable or power cable.

- For an NVMe SSD, remove the drive carefully after removing its retaining screw.

- Start the computer and enter the BIOS/UEFI setup.

- Check whether the storage drive is detected.

- Record the error message or BIOS indication.

- Shut down the computer again.

- Reconnect the SATA/NVMe drive properly.

- Start the computer and check BIOS/UEFI again.

- Confirm whether the drive is detected and whether the operating system boots.

### Before Reconnecting:

BIOS Indication/Error:
“Storage device not detected” / “No bootable device found” (depending on the system)

### After Reconnecting:

BIOS Indication:
The SSD/HDD is detected and the computer can normally continue the boot process.

### Result:

Disconnecting the storage device can cause the BIOS to report that no bootable device is available. After reconnecting the drive correctly, the BIOS should detect it again.

# Task 4: PC Randomly Restarts

### Component I Would Check First: RAM

I would check the RAM first because faulty, loose, or poorly seated RAM can cause system instability, crashes, freezes, and random restarts. It is also relatively easy to inspect and reseat.

### Diagnostic Steps:

Step 1 – Power Off

Turn off the PC and disconnect the power cable.

Step 2 – Check RAM

Open the case and locate the RAM modules. Check whether they are properly seated in their slots.

Step 3 – Reseat RAM

Remove the RAM carefully and reinstall it firmly until the locking clips are secured.

Step 4 – Test RAM Modules Individually

If there are multiple RAM sticks, test one stick at a time to identify a faulty module.

Step 5 – Test Different RAM Slots

If necessary, move the RAM to another compatible slot and test the system again.

Step 6 – Confirm the Fix

Use the computer normally and check whether the random restarting problem has stopped.