# Assignment: Wireless Network Configuration and Security

## 1. Set Up a Basic Wireless Network

### Objective

To create a basic wireless network using a home Wi-Fi router and connect a laptop or mobile phone to it.

### Network Topology

Internet

|

Wi-Fi Router

/            \

/              \

Laptop            Mobile

Wi-Fi              Wi-Fi

### Steps

- Connect the Wi-Fi router to a power source.

- Turn on the router.

- Open the wireless/network settings on the laptop or mobile phone.

- Search for available Wi-Fi networks.

- Find the SSID of the home Wi-Fi router.

- Enter the Wi-Fi password and connect to the network.

- Check the wireless settings of the router to find the current channel.

### Observation

| Item | Observed Value |
| --- | --- |
| Router SSID | __________________ |
| Wi-Fi Channel | __________________ |
| Security Type | __________________ |
| Connected Device | __________________ |

### Result

The laptop/mobile successfully connected to the wireless network using the router's SSID and password.

# 2. Change Wi-Fi Channel and Check Interference

### Objective

To change the Wi-Fi channel from Auto to a manually selected channel and check whether the new channel has less interference.

### Steps

- Log in to the Wi-Fi router's administration page.

- Open the Wireless/Wi-Fi Settings.

- Find the Channel option.

- Change the setting from Auto to a manual channel.

- For 2.4 GHz Wi-Fi, commonly used non-overlapping channels are:

  - Channel 1

  - Channel 6

  - Channel 11

- Save the settings.

- Use a Wi-Fi analyzer app on the phone to scan nearby Wi-Fi networks.

- Check which channels are being used by nearby networks.

- Select a channel with less overlap/interference.

- Reconnect the phone or laptop to the Wi-Fi network.

### Observation Table

| Test | Channel | Nearby Networks / Interference | Result |
| --- | --- | --- | --- |
| Before change | Auto | __________________ | __________________ |
| After change | 1 / 6 / 11 | __________________ | __________________ |

### Example

If many nearby networks are using Channel 6, but fewer networks are using Channel 1, Channel 1 may be a better choice.

### Result

The Wi-Fi channel was changed from Auto to a manually selected channel based on the observed nearby wireless interference.

# 3. Enable WPA2 or WPA3 Security

### Objective

To secure the wireless network using WPA2 or WPA3 and verify that only users with the correct password can connect.

### Steps

- Open the router's administration page.

- Go to Wireless Security settings.

- Select WPA2-Personal or WPA3-Personal, depending on router support.

- Enter a strong Wi-Fi password.

- Save the configuration.

- Disconnect the laptop/mobile from Wi-Fi.

- Try connecting again using the new password.

- Verify that the device connects successfully with the correct password.

- Try an incorrect password to confirm that unauthorized connection is rejected.

### Example Strong Password

Do not use this exact example for your real Wi-Fi:

Example: Net@Work2026!Secure

A good password should contain a combination of uppercase letters, lowercase letters, numbers, and special characters.

### Observation

| Test | Password | Result |
| --- | --- | --- |
| Correct password | Correct credentials | Connected |
| Wrong password | Incorrect credentials | Connection rejected |

### Result

WPA2/WPA3 security was successfully enabled, and the device could connect only when the correct credentials were provided.

# 4. Configure MAC Filtering

### Objective

To allow only selected devices to connect to the Wi-Fi network using MAC address filtering.

### What is a MAC Address?

A MAC address is a hardware-level network identifier associated with a network interface. In this practical, the router uses the MAC address to decide whether a device is allowed to connect.

### Steps

- Find the MAC address of your laptop or phone.

- Open the router administration page.

- Go to MAC Filtering / Access Control / Wireless Access Control.

- Enable MAC filtering.

- Add the MAC address of the authorized device to the allowed list.

- Save the settings.

- Connect the authorized device to Wi-Fi.

- Try connecting another device whose MAC address is not in the allowed list.

- Observe the result.

### Observation Table

| Device | MAC Address | Allowed? | Result |
| --- | --- | --- | --- |
| Authorized Device | __________________ | Yes | Connected |
| Unauthorized Device | __________________ | No | Blocked / Connection rejected |

### Result

The authorized device was allowed to connect, while a device whose MAC address was not included in the allowed list was blocked.

### Important Note

MAC filtering is an additional access-control feature, but it should not be treated as strong security by itself because MAC addresses can potentially be changed/spoofed. WPA2/WPA3 with a strong password should remain enabled.

# 5. Configure an ACL to Block YouTube

### Objective

To create an Access Control List (ACL) that blocks access to a specific website such as YouTube from the Wi-Fi network.

### Important Note

Blocking a modern website such as YouTube using a simple router ACL can be more complicated than blocking a single IP address because large services can use multiple IP addresses and HTTPS. Therefore, the exact method depends on the router model and features.

For a Packet Tracer lab, the following is a simple example using an extended ACL.

### Example Network

Wi-Fi Router

|

Wi-Fi Network

/          \

Laptop       Phone

|

Internet

|

YouTube

### Example ACL Configuration

Suppose the router uses an extended ACL and the lab provides a specific destination IP address for the website.

enable

configure terminal

access-list 100 deny ip 192.168.1.0 0.0.0.255 <YouTube-IP> 0.0.0.0

access-list 100 permit ip any any

exit

Apply the ACL to the appropriate interface:

interface g0/0

ip access-group 100 out

exit

Note: Replace <YouTube-IP> with the actual destination IP used in your lab. Do not assume that one fixed IP represents all of YouTube.

### Verify the ACL

Use:

show access-lists

You can check whether packets are matching the deny rule.

### Testing

- Connect the laptop/phone to the Wi-Fi network.

- Open a web browser.

- Try to access:

www.youtube.com

- Check whether the connection is blocked.

- Test another website to make sure general Internet access still works.

### Expected Observation

| Website | Expected Result |
| --- | --- |
| YouTube | Blocked |
| Other websites | Allowed |

### Result

The ACL was configured to deny traffic to the specified destination while allowing other traffic. The result depends on the router's ACL capabilities and the destination IP addresses used by the website.

# Overall Observation

| Practical | Main Result |
| --- | --- |
| Wireless Network | Device connected to Wi-Fi |
| Channel Selection | Channel selected based on interference |
| WPA2/WPA3 | Wireless network secured |
| MAC Filtering | Unauthorized device blocked |
| ACL | Specified website traffic blocked |