# WAN Authentication – Assignment

## 1. WAN Link Between Two Routers

Draw the following diagram:

LAN 1                              LAN 2

|                                  |

+------+                           +------+

| PC 1 |                           | PC 2 |

+------+                           +------+

|                                  |

+--------+       WAN Link          +--------+

| Router1|=========================| Router2|

+--------+      Serial Cable       +--------+

🔐 AUTHENTICATION

PAP / CHAP is required

on the WAN link

### Explanation

Two routers can communicate through a WAN serial link. Authentication such as PAP or CHAP can be configured on the WAN link to verify that the connecting router is authorized.

HDLC does not provide authentication, so it cannot verify the identity of the remote router.

# 2. HDLC WAN Connection

### Topology

Router1 ---------------- Serial WAN ---------------- Router2

G0/0: 192.168.1.1                              G0/0: 192.168.2.1

S0/0/0: 10.0.0.1 -------------------------- S0/0/0: 10.0.0.2

### Router1 Configuration

Router> enable

Router# configure terminal

Router(config)# interface serial 0/0/0

Router(config-if)# ip address 10.0.0.1 255.255.255.252

Router(config-if)# encapsulation hdlc

Router(config-if)# clock rate 64000

Router(config-if)# no shutdown

Router(config-if)# exit

### Router2 Configuration

Router> enable

Router# configure terminal

Router(config)# interface serial 0/0/0

Router(config-if)# ip address 10.0.0.2 255.255.255.252

Router(config-if)# encapsulation hdlc

Router(config-if)# no shutdown

Router(config-if)# exit

Note: clock rate is normally configured on the DCE side of a serial connection. In Packet Tracer, check which router interface is DCE.

### Test Connection

From Router1:

Router# ping 10.0.0.2

If the configuration is correct, the ping should succeed.

### Why HDLC Does Not Provide Authentication?

HDLC (High-Level Data Link Control) provides data-link encapsulation and framing, but it does not have built-in username/password authentication.

There is no command such as:

ppp authentication pap

or

ppp authentication chap

when using HDLC.

Therefore, HDLC cannot authenticate the identity of the other router.

# 3. PPP with PAP Authentication

First change the encapsulation from HDLC to PPP.

### Router1

Router1> enable

Router1# configure terminal

Router1(config)# username Router2 password cisco

Router1(config)# interface serial 0/0/0

Router1(config-if)# ip address 10.0.0.1 255.255.255.252

Router1(config-if)# encapsulation ppp

Router1(config-if)# ppp authentication pap

Router1(config-if)# ppp pap sent-username Router1 password cisco

Router1(config-if)# no shutdown

### Router2

Router2> enable

Router2# configure terminal

Router2(config)# username Router1 password cisco

Router2(config)# interface serial 0/0/0

Router2(config-if)# ip address 10.0.0.2 255.255.255.252

Router2(config-if)# encapsulation ppp

Router2(config-if)# ppp authentication pap

Router2(config-if)# ppp pap sent-username Router2 password cisco

Router2(config-if)# no shutdown

### Test

Router1# ping 10.0.0.2

If the username and password match, the PPP connection should come UP.

### What if the Password Is Incorrect?

If the password is incorrect, the receiving router rejects the authentication. As a result, the PPP link will not successfully establish communication.

Simple example:

Router1 password: cisco

Router2 password: abc123

Because the passwords don't match, authentication fails and the PPP connection will not work correctly.

### PAP in Simple Words

PAP = Username + Password authentication

The credentials are sent as part of the authentication process, so PAP provides weaker security than CHAP.

# 4. PPP with CHAP Authentication

Now replace PAP with CHAP.

### Router1

Router1(config)# username Router2 password cisco

Router1(config)# interface serial 0/0/0

Router1(config-if)# encapsulation ppp

Router1(config-if)# ppp authentication chap

Router1(config-if)# no shutdown

### Router2

Router2(config)# username Router1 password cisco

Router2(config)# interface serial 0/0/0

Router2(config-if)# encapsulation ppp

Router2(config-if)# ppp authentication chap

Router2(config-if)# no shutdown

### Test

Router1# ping 10.0.0.2

### PAP vs CHAP

| Feature | PAP | CHAP |
| --- | --- | --- |
| Authentication | Username/password | Challenge-response |
| Security | Lower | Higher |
| Password sent directly | Yes | No |
| Re-authentication | No | Can periodically re-authenticate |

### Why CHAP Is More Secure

CHAP does not send the actual password across the link. Instead, it uses a challenge-response mechanism involving a one-way hash.

So:

PAP → sends credentials → weaker security

CHAP → challenge + response → better security

# 5. Real-World WAN Authentication Examples

## Example 1: Mobile Data Authentication

Mobile networks authenticate subscribers before allowing them to access mobile data services.

Mobile Phone

|

↓

Cell Tower

|

↓

Mobile Network

|

↓

Internet

Authentication ensures that only valid subscribers/SIMs can use the network. It helps prevent unauthorized users from accessing network services and protects subscriber accounts.

## Example 2: Public Wi-Fi Login

Many public Wi-Fi networks require users to authenticate before getting Internet access.

Laptop / Phone

|

↓

Wi-Fi Access Point

|

↓

Router / WAN

|

↓

Internet

Authentication prevents unauthorized users from freely accessing the network. It can also allow the network provider to control access and associate network usage with an authenticated account.

Note: Public Wi-Fi commonly uses web-based captive portals or WPA2/WPA3-Enterprise mechanisms; PAP/CHAP are examples of authentication methods used in some underlying access systems, rather than being universal public-Wi-Fi protocols.