# Assignment: OSPF Configuration and Adjacency in Cisco Packet Tracer

## 1. Create a Three-Router Triangle Topology

### Objective

To create a network with three routers connected in a triangle topology using Ethernet or serial links.

### Network Topology

R1

/  \

/    \

/      \

R2------R3

The three routers are connected directly to each other.

- R1 is connected to R2.

- R2 is connected to R3.

- R3 is connected to R1.

This creates a triangle topology.

### IP Addressing Table

| Router | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0 | 10.0.12.1 | 255.255.255.252 |
| R1 | G0/1 | 10.0.13.1 | 255.255.255.252 |
| R2 | G0/0 | 10.0.12.2 | 255.255.255.252 |
| R2 | G0/1 | 10.0.23.1 | 255.255.255.252 |
| R3 | G0/0 | 10.0.13.2 | 255.255.255.252 |
| R3 | G0/1 | 10.0.23.2 | 255.255.255.252 |

### Network Diagram with IP Addresses

R1

10.0.12.1

/       \

/         \

10.0.12.2           10.0.13.2

R2 -------- R3

10.0.23.1   10.0.23.2

# 2. Assign IP Addresses and Verify Connectivity

## Router 1 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.13.1 255.255.255.252

no shutdown

exit

## Router 2 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

## Router 3 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.13.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

## Verify Interfaces

Use:

show ip interface brief

All connected interfaces should show:

Status: up

Protocol: up

## Test Connectivity

From R1:

ping 10.0.12.2

ping 10.0.13.2

From R2:

ping 10.0.12.1

ping 10.0.23.2

From R3:

ping 10.0.13.1

ping 10.0.23.1

If the configuration is correct, the ping should receive replies.

### Result

The ping test confirms that all three routers can communicate with their directly connected neighbors.

# 3. Configure OSPF on All Routers

### Objective

To configure OSPF process 1 on all three routers and advertise all directly connected networks.

OSPF stands for Open Shortest Path First.

## Router 1

enable

configure terminal

router ospf 1

router-id 1.1.1.1

network 10.0.12.0 0.0.0.3 area 0

network 10.0.13.0 0.0.0.3 area 0

exit

## Router 2

enable

configure terminal

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

exit

## Router 3

enable

configure terminal

router ospf 1

router-id 3.3.3.3

network 10.0.13.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

exit

### Important Commands

show ip ospf

This command displays general OSPF information.

show ip route ospf

This command displays routes learned through OSPF.

OSPF routes are marked with:

O

Example:

O    10.0.23.0/30 [110/2] via 10.0.12.2

Here:

- O = OSPF route

- 110 = OSPF administrative distance

- 2 = OSPF cost/metric

# 4. Observe OSPF Adjacency Formation

### Objective

To observe the OSPF neighbor relationship between the three routers.

Use the following command on each router:

show ip ospf neighbor

### Expected Output

For example, R1 may show:

Neighbor ID     State

2.2.2.2         FULL

3.3.3.3         FULL

R2 may show:

Neighbor ID     State

1.1.1.1         FULL

3.3.3.3         FULL

R3 may show:

Neighbor ID     State

1.1.1.1         FULL

2.2.2.2         FULL

### OSPF Neighbor States

OSPF neighbors go through different states while forming an adjacency.

Important states include:

- Down

- Init

- 2-Way

- ExStart

- Exchange

- Loading

- FULL

The FULL state means the OSPF routers have successfully formed an adjacency and their link-state databases are synchronized.

### Observation Table

| Router | Neighbor | Expected State |
| --- | --- | --- |
| R1 | R2 | FULL |
| R1 | R3 | FULL |
| R2 | R1 | FULL |
| R2 | R3 | FULL |
| R3 | R1 | FULL |
| R3 | R2 | FULL |

### Result

The show ip ospf neighbor command confirms that the three routers have successfully formed OSPF adjacencies.

# 5. Change OSPF Cost and Observe the Routing Table

### Objective

To change the OSPF cost of an interface and observe how OSPF selects a different path.

Suppose we want to increase the cost of the direct link between R1 and R3.

On R1:

enable

configure terminal

interface g0/1

ip ospf cost 100

exit

The cost of this interface is now 100.

### Verify the OSPF Route

Use:

show ip route ospf

Before changing the cost, OSPF may prefer the direct path:

R1 → R3

After increasing the cost, OSPF may prefer:

R1 → R2 → R3

because the alternate path has a lower total OSPF cost.

### Example

Before:

R1 -------- R3

\          /

\        /

\-- R2 -/

Preferred path:

R1 → R3

After increasing R1-R3 cost:

R1 -------- R3

\

\

R2 -------- R3

Preferred path:

R1 → R2 → R3

### Verification Commands

Check the routing table:

show ip route ospf

Check the interface:

show ip ospf interface g0/1

The output will show the configured OSPF cost.

### Observation Table

| Condition | Preferred Path | OSPF Cost |
| --- | --- | --- |
| Before cost change | R1 → R3 | Lower |
| After increasing R1-R3 cost | R1 → R2 → R3 | Lower total cost |

### Result

Changing the OSPF cost can change the preferred path selected by OSPF. OSPF chooses the path with the lowest total cost.