### Day 2: Diagnosing Hardware Failures

Task 1: Five Common Hardware Issues

| Hardware Issue | Typical Symptom |
| --- | --- |
| 1. RAM Failure | PC may show no display, beep codes, or may restart repeatedly. |
| 2. SMPS/Power Supply Failure | PC does not turn ON or suddenly shuts down. |
| 3. Hard Disk/SSD Failure | Computer may become very slow or show boot errors. |
| 4. CPU Overheating | PC may freeze, restart, or shut down automatically. |
| 5. Graphics Card Failure | No display, screen artifacts, or display driver-related hardware symptoms. |

### Task 2: Laptop Not Charging – Troubleshooting Plan

- Check the power socket – Connect another device to make sure the socket is working.

- Inspect the charger and cable – Look for cuts, burns, loose connections, or physical damage.

- Check the charger output – Use a multimeter to measure the DC output voltage of the charger. Compare it with the voltage printed on the charger.

- Check the charging connector – Make sure the laptop's charging port is not loose or physically damaged.

- Check battery connections – If safely accessible, inspect whether the battery connector is properly connected.

- Test with another compatible charger – If available, use a known-good compatible charger to determine whether the original charger is faulty.

- Identify the faulty component – Based on the tests, the problem may be with the charger, charging port, battery, or charging circuit.

Safety: Never short the charger pins or open a mains-powered charger.

### Task 3: Testing a USB Phone Charger

Tool Used: Digital Multimeter

Expected USB Voltage: 4.75V – 5.25V

Procedure:

- Set the multimeter to DC Voltage (V⎓).

- Set the range above 5V if the meter is not auto-ranging.

- Connect the USB charger to a power outlet.

- Identify the USB 5V (VBUS) and GND contacts.

- Touch the red probe to the 5V contact and the black probe to GND.

- Record the voltage shown on the multimeter.

Example Reading: 5.08V

Result: 5.08V is within the expected range of 4.75V–5.25V, so the charger output is normal.

Important: If you have not actually measured your charger, don't submit 5.08V as your real measurement. Replace it with your actual reading.

### Task 4: Testing the ATX 12V Rail

Tool Used: Digital Multimeter

Expected 12V Rail: Approximately 12V
Yellow wire: +12V
Black wire: Ground (GND)

Procedure:

- Shut down the PC and disconnect the AC power cable.

- Disconnect the ATX power connector from the motherboard.

- Set the multimeter to DC Voltage (V⎓).

- Turn the PC on only if performing an appropriate controlled PSU test; otherwise use a proper PSU tester or qualified technician.

- Place the black probe on the black GND wire.

- Place the red probe on the yellow +12V wire.

- Read and record the voltage shown on the multimeter.

- Compare the reading with the expected 12V value.

Example Reading: 12.08V

Result: 12.08V is close to 12V and is not below 11.5V.

If the voltage is below 11.5V:
It could indicate a faulty or weak power supply (SMPS/PSU) or a power-delivery problem. This can contribute to random shutdowns.