# Assignment: OSPF Multi-Area Configuration in Cisco Packet Tracer

## 1. Create a Three-Router OSPF Multi-Area Topology

### Objective

To create a network topology with three routers using two OSPF areas: Area 0 and Area 1.

- Area 0 is the OSPF backbone area.

- Area 1 is connected to Area 0 through Router2.

- Router2 acts as an ABR (Area Border Router) because it connects Area 0 and Area 1.

### Network Topology

AREA 0

Backbone Area

PC1

|

Switch

|

R1

|

| 10.0.12.0/30

|

R2

Area 0 / Area 1

|

| 10.0.23.0/30

|

R3

|

Switch

|

PC2

AREA 1

### IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | OSPF Area |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | Area 0 |
| R1 | G0/1 | 10.0.12.1 | 255.255.255.252 | Area 0 |
| R2 | G0/0 | 10.0.12.2 | 255.255.255.252 | Area 0 |
| R2 | G0/1 | 10.0.23.1 | 255.255.255.252 | Area 1 |
| R3 | G0/0 | 10.0.23.2 | 255.255.255.252 | Area 1 |
| R3 | G0/1 | 192.168.2.1 | 255.255.255.0 | Area 1 |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 | Area 0 |
| PC2 | NIC | 192.168.2.10 | 255.255.255.0 | Area 1 |

# 2. Configure OSPF with Area 0 and Area 1

## Router 1 Configuration

Router1 is completely inside Area 0.

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

exit

## Router 2 Configuration

Router2 connects Area 0 and Area 1, so it acts as an ABR.

enable

configure terminal

interface g0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 1

exit

## Router 3 Configuration

Router3 is inside Area 1.

enable

configure terminal

interface g0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 3.3.3.3

network 10.0.23.0 0.0.0.3 area 1

network 192.168.2.0 0.0.0.255 area 1

exit

### OSPF Area Design

AREA 0

┌──────────────┐

│              │

R1 ------------ R2

|

|

AREA 1

|

|

R3

Router2 is the ABR because it has interfaces in both Area 0 and Area 1.

# Verify OSPF Neighbor Relationships

On each router, use:

show ip ospf neighbor

### Expected Result

On R1:

Neighbor ID     State

2.2.2.2         FULL

On R2:

Neighbor ID     State

1.1.1.1         FULL

3.3.3.3         FULL

On R3:

Neighbor ID     State

2.2.2.2         FULL

The FULL state confirms that OSPF neighbors have successfully formed an adjacency.

### Observation Table

| Router | Neighbor | Area | State |
| --- | --- | --- | --- |
| R1 | R2 | Area 0 | FULL |
| R2 | R1 | Area 0 | FULL |
| R2 | R3 | Area 1 | FULL |
| R3 | R2 | Area 1 | FULL |

# 3. Add PCs and Test Connectivity

## PC1 Configuration

PC1 belongs to Area 0.

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

## PC2 Configuration

PC2 belongs to Area 1.

IP Address: 192.168.2.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

### Test Connectivity

Open the Command Prompt on PC1 and enter:

ping 192.168.2.10

If OSPF is configured correctly, PC1 should receive replies from PC2.

Example:

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

### Test from PC2

ping 192.168.1.10

PC2 should also receive replies from PC1.

### Result

The successful ping confirms that devices in Area 0 and Area 1 can communicate through the OSPF backbone and ABR.

# 4. Simulate Router/Link Failure and OSPF Reconvergence

## Objective

To simulate a failure between Area 0 and Area 1 and observe how OSPF updates its routing table.

The connection between R2 and R3 is the link between Area 0 and Area 1.

### Step 1: Check the Routing Table Before Failure

On R1:

show ip route

Look for the route:

192.168.2.0/24

It should be learned through OSPF.

You can also use:

show ip route ospf

### Step 2: Check OSPF Information

Use:

show ip ospf

This displays information about the OSPF process, areas, and link-state information.

## Step 3: Shut Down the Area 1 Connection

On Router2:

enable

configure terminal

interface g0/1

shutdown

The connection between R2 and R3 will go down.

### Step 4: Observe the Routing Table

Immediately check Router1:

show ip route

Then check:

show ip ospf

The OSPF information and routing table will change after the network detects the failure and reconverges.

# Record the Reconvergence Time

Use a stopwatch to record the approximate time.

| Event | Time |
| --- | --- |
| Interface shutdown | __________ |
| Failure detected | __________ |
| Route removed/updated | __________ |
| Total reconvergence time | __________ |

Note: The exact time can vary depending on the Cisco Packet Tracer version, OSPF timers, topology, and simulation conditions. Do not assume a fixed time; record the value observed in your own Packet Tracer experiment.

# Step 5: Restore the Connection

On Router2:

configure terminal

interface g0/1

no shutdown

Wait for OSPF to establish the neighbor relationship again.

Check:

show ip ospf neighbor

The R2-R3 neighbor should eventually return to:

FULL

Then check:

show ip route

and:

show ip route ospf

The route to the Area 1 network should be learned again.

# Recovery Observation Table

| Event | Observation |
| --- | --- |
| Before failure | R1 can reach PC2 |
| R2-R3 interface shutdown | Area 1 connection fails |
| OSPF detects failure | Routing information changes |
| Route removed/updated | Area 1 route changes |
| Interface re-enabled | OSPF neighbor reforms |
| Neighbor state | FULL |
| Connectivity restored | PC1 can ping PC2 |

# Important OSPF Concepts

### Area 0

Area 0 is called the OSPF backbone area. Other OSPF areas normally connect to the backbone.

### Area 1

Area 1 is a separate OSPF area connected to Area 0 through Router2.

### ABR

ABR = Area Border Router

Router2 is an ABR because it connects:

Area 0 ←→ Router2 ←→ Area 1

### OSPF Reconvergence

When a link fails, OSPF detects the change, exchanges updated link-state information, recalculates the best path, and updates the routing table. This process is called OSPF reconvergence.