Name – parikh marmik

- Addressing and Subnetting

# Ans=

| Class | Range | Default Subnet Mask | Real-World Example |
| --- | --- | --- | --- |
| A | 1 – 126 | 255.0.0.0 | Large company or ISP |
| B | 128 – 191 | 255.255.0.0 | College or University |
| C | 192 – 223 | 255.255.255.0 | Home WiFi Router or Small Office |
| D | 224 – 239 | Not Applicable | Multicast (IPTV, Live Streaming) |
| E | 240 – 255 | Not Applicable | Research and Experimental |

## IP Address Calculation

Ans=

- IP Address = 192.168.10.45

- Subnet Mask = 255.255.255.0 (/24)

### Step 1: Network Address

Network Address = 192.168.10.0

### Step 2: Broadcast Address

Broadcast Address = 192.168.10.255

### Step 3: Valid Host Range

First Host = 192.168.10.1

Last Host = 192.168.10.254

3. Subnetting Plan (10.0.0.0/24)

### Ans= Given Network

10.0.0.0/24

Need 4 Subnets

Borrow 2 bits

New Prefix = /26

Subnet Mask = 255.255.255.192

- College WiFi Network Subnetting

Ans=

Network = 172.16.0.0/16

Required = 20 Subnets

### Step 1

2⁴ = 16

2⁵ = 32

Borrow = 5 bits

### Step 2

Original Prefix = /16

New Prefix = /21

### Step 3

New Subnet Mask = 255.255.248.0

### Step 4

Number of Subnets = 32

### Step 5

Remaining Host Bits = 11

Hosts = 2¹¹ = 2048

Usable Hosts = 2046

# 5. Invalid IP Address Explanation

Ans=

The IP address 192.168.1.300 is invalid because each octet in an IPv4 address must be between 0 and 255.

Here, the last octet is 300, which is not allowed.

For the subnet 192.168.1.0/24:

- Network Address = 192.168.1.0

- Broadcast Address = 192.168.1.255

- Valid Host Range = 192.168.1.1 to 192.168.1.254

Examples of valid IP addresses:

- 192.168.1.10

- 192.168.1.50

- 192.168.1.100

- 192.168.1.200