# Assignment: Static Routing vs Dynamic Routing and RIP

## 1. Three Differences Between Static Routing and Dynamic Routing

| Point | Static Routing | Dynamic Routing | Real-World Analogy |
| --- | --- | --- | --- |
| 1. Route Configuration | Routes are manually configured by the network administrator. | Routes are learned automatically using routing protocols such as RIP, OSPF, or EIGRP. | Google Maps: Static routing is like manually telling a driver which road to take. Dynamic routing is like Google Maps automatically selecting a route. |
| 2. Network Changes | Static routes do not automatically change when a link fails. | Dynamic routing can learn about network changes and update routing information. | Swiggy: If one road is closed, dynamic routing is like Swiggy automatically finding another available path for delivery. |
| 3. Suitable Network Size | Best for small and stable networks. | Better for large and changing networks. | Google Maps: Manually planning routes works for a small trip, but automatic route calculation is better when there are many roads and traffic changes. |

### Summary

Static routing is simple and predictable but requires manual configuration. Dynamic routing automatically learns and updates routes, making it more suitable for large networks.

# 2. Observe RIP Routes Using Cisco Packet Tracer

## Objective

To observe RIP-learned routes and their metrics using the show ip route and show ip protocols commands on a three-router RIP network.

## Network Topology

PC1 ── R1 ── R2 ── R3 ── PC3

|      |      |

RIP    RIP    RIP

The three routers are configured to use RIP.

### Example IP Networks

| Router | Network |
| --- | --- |
| R1 | 192.168.1.0/24 |
| R1-R2 | 10.0.12.0/30 |
| R2 | 192.168.2.0/24 |
| R2-R3 | 10.0.23.0/30 |
| R3 | 192.168.3.0/24 |

## Step 1: Open the Packet Tracer File

- Open Cisco Packet Tracer.

- Open the pre-configured RIP network file.

- Make sure all three routers are powered on.

- Wait for the RIP routes to be learned.

## Step 2: Check the Routing Table

On Router1, enter:

enable

show ip route

Look for routes marked with:

R

R means the route was learned through RIP.

### Example

R    192.168.2.0/24 [120/1] via 10.0.12.2

R    192.168.3.0/24 [120/2] via 10.0.12.2

Here:

- R = RIP-learned route

- 120 = RIP administrative distance

- 1 or 2 = RIP metric (hop count)

- via = Next-hop router

## Step 3: Check RIP Protocol Information

Enter:

show ip protocols

This command displays information about the routing protocol running on the router.

It can show:

- RIP is enabled.

- Networks advertised by RIP.

- RIP version.

- Routing update information.

- RIP timers.

## Observation Table

Fill in the actual values displayed by your Packet Tracer file.

| Router | RIP-Learned Network | Next Hop | Metric |
| --- | --- | --- | --- |
| R1 | 192.168.2.0/24 | 10.0.12.2 | 1 |
| R1 | 192.168.3.0/24 | 10.0.12.2 | 2 |
| R2 | 192.168.1.0/24 | 10.0.12.1 | 1 |
| R2 | 192.168.3.0/24 | 10.0.23.2 | 1 |
| R3 | 192.168.1.0/24 | 10.0.23.1 | 2 |
| R3 | 192.168.2.0/24 | 10.0.23.1 | 1 |

Note: The exact next-hop addresses and metrics may be different depending on the provided Packet Tracer file.

# 3. Simulate a Network Change in RIP

## Objective

To shut down one router interface and observe how RIP updates the routing tables.

### Step 1: Select an Interface

For example, on Router2:

enable

configure terminal

interface serial 0/0/0

shutdown

The interface will go down.

### Step 2: Check the Routing Table

Immediately check Router1:

show ip route

Repeat the command periodically and observe the RIP route.

Also check Router3:

show ip route

The route that depends on the failed link will eventually be removed or replaced.

### Step 3: Record the Time

Use a stopwatch or timer to record:

- Time when the interface was shut down.

- Time when the route disappeared from the routing table.

### Observation Table

| Event | Time |
| --- | --- |
| Interface shutdown | __________ |
| Route still visible | __________ |
| Route removed/updated | __________ |
| Total time taken | __________ |

### Important Note

RIP does not update the routing table instantly. It uses periodic routing updates and timers to detect unreachable routes. The exact time observed in Packet Tracer can vary depending on the topology, RIP configuration, and timers.

### Restore the Interface

After the test, restore the interface:

configure terminal

interface serial 0/0/0

no shutdown

Then check:

show ip route

The RIP route should be learned again after the network converges.

# 4. Dynamic Routing in Uber/Zomato and RIP

Dynamic routing in networks is similar to route planning in apps such as Uber or Zomato. In an app, the system can automatically update the path when traffic, road closures, or other conditions change. Similarly, RIP automatically exchanges routing information between routers and updates its routing table when network paths change. Therefore, both systems can automatically adapt to changes instead of requiring every route to be manually updated.